# Lab 12 – Worksheet

| Name:Sarosh mehdi, Sualeh abbas, Hamza hasan, Ghazi Hussain | ID:10314, 10303, 10346, 10345 | Section: T2 |
| --- | --- | --- |

### State Transition Diagram

Sketch the state transition diagram. Indicate the input conditions that cause
transitions among states. Use the signals ‘left’ and ‘right’ for the direction indicator.

### State Assignment

Along with your state transition diagram, pick any state assignment method to map symbolic states to binary values.

IDLE  000

L1  001

L2  010

L3  011

R1  101

R2  110

R3  111

### State Transition Table

Construct state transition table based on state diagram and chosen state assignment. Add columns for mapping the states to output.

| A | B | C | RES | R | L | AN | BN | CN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | X | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | X | X | X | 0 | 1 | 0 |
| 0 | 1 | 0 | X | X | X | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | X | X | X | 1 | 1 | 0 |
| 1 | 1 | 0 | X | X | X | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |

### Circuit Diagram of FSM

Obtain input and output equations and sketch circuit diagram to implement the FSM (a sample of Mealy FSM is assuming the D-flip flop as memory element is given in Figure 12.1 as a reference)

Figure 12.1 - A simple view of an FSM

Output Equations: LC=A’.B.C LB=A’.B LA=A’.C RA=A RB=A.C RC=A.B

### Verilog Implementation

On Eda Playground or Vivado create a new project Add a new Verilog source and implement the finite state machine using what you have learned in the lecture about Verilog. Be sure to include a clock input (clk) and a reset (rst) in your circuit.

Provide appropriately commented code here

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Module Name: tail_light_fsm

// Description: Moore FSM to control 1965 Ford Thunderbird tail lights

//////////////////////////////////////////////////////////////////////////////////

module tail_light_fsm (

input clk,

input reset,

input left,

input right,

input enable,              // Enable signal from clock divider

output reg LC, LB, LA,

output reg RA, RB, RC

);

// State encoding

parameter IDLE = 3'b000,

L1   = 3'b001,

L2   = 3'b010,

L3   = 3'b011,

R1   = 3'b100,

R2   = 3'b101,

R3   = 3'b110;

reg [2:0] state, next_state;

// State register with synchronous reset

always @(posedge clk or posedge reset) begin

if (reset)

state <= IDLE;

else if (enable)

state <= next_state;

end

// Next-state logic

always @(*) begin

case (state)

IDLE: begin

if (left)

next_state = L1;

else if (right)

next_state = R1;

else

next_state = IDLE;

end

L1:  next_state = L2;

L2:  next_state = L3;

L3:  next_state = IDLE;

R1:  next_state = R2;

R2:  next_state = R3;

R3:  next_state = IDLE;

default: next_state = IDLE;

endcase

end

// Output logic (Moore: depends only on state)

always @(*) begin

// Default off

LA = 0; LB = 0; LC = 0;

RA = 0; RB = 0; RC = 0;

case (state)

L1: LA = 1;

L2: begin LA = 1; LB = 1; end

L3: begin LA = 1; LB = 1; LC = 1; end

R1: RA = 1;

R2: begin RA = 1; RB = 1; end

R3: begin RA = 1; RB = 1; RC = 1; end

default: ; // IDLE or invalid state

endcase

end

endmodule

### Implementing the Clock

The clock available on Basys-3 board is 100 MHz, means that every clock period is 10 nanoseconds long. The entire blinking sequence would be then 40 ns. This is a very short time. light travels less than 250 meters during that time. If we want to see our sequence, we need to find a way to slow down the circuit. This can be achieved in two ways. We can either implement a clock divider circuit that divides the clock by a few million times, or we can generate an enable signal every few million cycles, and then use this enable signal to control our next state transition. Refer to Lab 10 to design a clock divider of your desired value

Provide appropriately commented code of clock div

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Module Name: clock_divider

// Description: Divides 100MHz Basys-3 clock to a slower pulse for FSM transitions

//////////////////////////////////////////////////////////////////////////////////

module clock_divider #(parameter COUNT_MAX = 100_000_000)(

input clk,

input reset,

output reg enable

);

reg [26:0] counter = 0;

always @(posedge clk or posedge reset) begin

if (reset) begin

counter <= 0;

enable <= 0;

end else if (counter == COUNT_MAX - 1) begin

counter <= 0;

enable <= 1; // Send a single-cycle enable pulse

end else begin

counter <= counter + 1;

enable <= 0;

end

end

endmodule

### Top Level Module

Write top module to implement FSM along with clock divider

Provide appropriately commented code for designed module

// Top-level module for controlling tail lights on a Basys-3 board

module top_module (

input clk,         // 100MHz clock input from the Basys-3 board

input reset,       // Active-high reset signal

input left,        // Signal to activate left turn lights

input right,       // Signal to activate right turn lights

output LC, LB, LA, // Outputs for left turn lights (Center, B, A)

output RA, RB, RC  // Outputs for right turn lights (A, B, Center)

);

wire enable;       // Enable signal generated by the clock divider to control the FSM speed

// Instantiate clock divider

// This module divides the 100MHz clock to a lower frequency to slow down the FSM transitions,

// making the tail light effect visible to the human eye

clock_divider #(100_000_000) clkdiv (

.clk(clk),        // Input clock (100 MHz)

.reset(reset),    // Reset signal

.enable(enable)   // Output enable signal (slower clock pulse)

);

// Instantiate the finite state machine (FSM) for tail lights

// This FSM handles the light sequencing based on the left/right input

tail_light_fsm fsm (

.clk(clk),        // Input clock

.reset(reset),    // Reset signal

.left(left),      // Input to activate left light sequence

.right(right),    // Input to activate right light sequence

.enable(enable),  // Enables FSM state transition when high

// Outputs to control each individual tail light

.LC(LC), .LB(LB), .LA(LA),  // Left side tail lights

.RA(RA), .RB(RB), .RC(RC)   // Right side tail lights

);

endmodule

### Define the constraints (to create .xdc file)

Define constrains for your module to implement on Basys-3 Board

Add Constraint file here

set_property PACKAGE_PIN W5 [get_ports clk]

set_property PACKAGE_PIN L1 [get_ports LA]

set_property PACKAGE_PIN P1 [get_ports LB]

set_property PACKAGE_PIN N3 [get_ports LC]

set_property PACKAGE_PIN U19 [get_ports RA]

set_property PACKAGE_PIN E19 [get_ports RB]

set_property PACKAGE_PIN U16 [get_ports RC]

set_property PACKAGE_PIN U18 [get_ports reset]

set_property PACKAGE_PIN W19 [get_ports left]

set_property PACKAGE_PIN T17 [get_ports right]

set_property IOSTANDARD LVCMOS33 [get_ports clk]

set_property IOSTANDARD LVCMOS33 [get_ports LA]

set_property IOSTANDARD LVCMOS33 [get_ports LB]

set_property IOSTANDARD LVCMOS33 [get_ports LC]

set_property IOSTANDARD LVCMOS33 [get_ports left]

set_property IOSTANDARD LVCMOS33 [get_ports RA]

set_property IOSTANDARD LVCMOS33 [get_ports RB]

set_property IOSTANDARD LVCMOS33 [get_ports RC]

set_property IOSTANDARD LVCMOS33 [get_ports reset]

set_property IOSTANDARD LVCMOS33 [get_ports right]

### Test on BASYS 03 Board

Program Basys-3 board to test and validate your designed modules

RA Signature

## Assessment Rubric

Marks Distribution:

|  |  | LR2 Code/Model | LR5 Results | LR11 Design | LR7 Viva | LR9 Report |
| --- | --- | --- | --- | --- | --- | --- |
| In-Lab | Task 1 | 10 |  |  | 10 | 10 |
|  | Task 2 | 10 |  |  |  |  |
|  | Task 3 | 10 |  |  |  |  |
|  | Task 4 |  |  | 10 |  |  |
|  | Task 5 | 10 |  |  |  |  |
|  | Task 6 | 10 |  |  |  |  |
|  | Task 7 | 5 |  |  |  |  |
|  | Task 8 | 5 |  |  |  |  |
|  | Task 9 |  | 10 |  |  |  |
| Total |  |  |  |  |  | 100 Points |

Marks Obtained:

|  |  | LR2 Code/Model | LR5 Results | LR11 Design | LR7 Viva | LR9 Report |
| --- | --- | --- | --- | --- | --- | --- |
| In-Lab | Task 1 |  |  |  |  |  |
|  | Task 2 |  |  |  |  |  |
|  | Task 3 |  |  |  |  |  |
|  | Task 4 |  |  |  |  |  |
|  | Task 5 |  |  |  |  |  |
|  | Task 6 |  |  |  |  |  |
|  | Task 7 |  |  |  |  |  |
|  | Task 8 |  |  |  |  |  |
|  | Task 9 |  |  |  |  |  |
| Total |  |  |  |  |  |  |