# Lab 04

# Fixed-Bias and Emitter-Bias BJT Biasing Configurations

| Name: Syed Sualeh Abbas, Syed Haider Zaidi | Student ID: 10303 |
| --- | --- |

## Objective

To investigate the effect of current gain β variation on quiescent operating conditions of a Bipolar Junction Transistor in fixed bias and emitter bias configurations.

Note: All the plots required to be attached should be labeled properly, followed by Task number and must be readable, i.e., use white background, suitable marker color (recommended – any dark color) and increase the width of trace. On PSpice there is an option to add text labels in the plots.

## Introduction to BJT Biasing

The term biasing refers to the application of DC voltages at transistor terminals to establish a fixed level of current and voltage. The operating point (IC, VCE) is a fixed point on the characteristics, it is also called the quiescent point (abbreviated Q -point). By definition, quiescent means quiet, still, inactive. The chosen Q -point often depends on the intended use of the circuit like application of transistor as amplifier or switch.

Since, temperature causes the device parameters such as the transistor current gain and the transistor leakage current (ICEO) to change. Higher temperatures result in increased leakage currents in the device, thereby changing the operating condition set by the biasing network. Therefore, the network design must also provide a degree of temperature stability so that temperature changes result in minimum changes in the operating point. This maintenance of the operating point can be specified by a stability factor S, which indicates the degree of change in operating point due to variation in parameters. A highly stable circuit is desirable. In this lab, two biasing circuits will be investigated and their stability will be compared. After that, you will be asked to design a biasing network for given operating point.

## Fixed Bias Configuration

The fixed-bias circuit of Figure is the simplest transistor DC bias configuration.

Figure 4.1: Fixed bias BJT configuration

Analysis of the circuit can be done by considering its two loops; the base-emitter loop and the collector-emitter loop. From base-emitter loop, the base current IB is given by;

The magnitude of collector current IC is related to the base current IB through . Then, collector-to-emitter voltage VCE is given by

Note that here, VE is 0 V as emitter is directly grounded.

### Task 1: To calculate the operating point of BJT in given fixed-bias configuration

Consider a BJT 2N3904 biased in fixed-bias configuration shown in Figure 4.1 with VCC being 20V, RB of 1 MΩ and RC of 2.7 kΩ. Using the given formulae, perform calculations to find the operating point of BJT. Use β = 140 and 0.7 V base-to-emitter forward voltage for the given transistor in your calculations.

Table 4.1: Calculated values of operating point of 2N3904 in given fixed bias configuration

| Parameter | Calculated Value |
| --- | --- |
| Base-to-Emitter Voltage: VBE | 0.7 V (Given) |
| Current Gain: β | 140 (Given) |
| Base Current: IB | 19.3uA |
| Collector Current: IC | 2.7mA |
| Collector-to-Emitter Voltage: VCE | 12.71 V |

### Task 2: To implement fixed-bias configuration circuit with 2N3904 and 2N4401 BJT.

- Construct the circuit of Figure 4.1 in with the component values as given in Task 1 for 2N3904 BJT model.

- Do not use a multimeter to identify terminals. Use the datasheet of 2N3904 for its pin configuration.

- Measure the parameters listed in Table 4.2 to find the operating point of BJT.

- Necessary precautions shall be ensured while using DMM Ammeter to avoid damage to the equipment. After measuring the current the probe is to be always put back to voltage section.

Table 4.2: Measured results of fixed-bias configuration with 2 different BJT models

| Parameter | Measured Results |  |
| --- | --- | --- |
|  | 2N3904 | 2N4401 |
| Base Current: IB | 22.41uA | 20uA |
| Collector Current: IC | 4.5mA | 5.09mA |
| Collector-to-Emitter Voltage: VCE | 7.7V | 6.0V |
| Current Gain: β (calculated) | 200.80 | 254.5 |
| Base-to-Emitter Voltage: VBE | 1.80V | 2.0V |
| Measured Resistances | 0.97M, 2.74K | 0.97M, 2.74K |

- Are the obtained results same as the calculated ones? What can be a possible reason behind discrepancies (if any)? Compare the value of current gain β.

The measured results show significant differences from the calculated values. For the 2N3904 transistor, the measured collector current (4.5mA) is notably higher than the calculated value (2.7mA), and the collector-to-emitter voltage (7.7V) is lower than calculated (12.71V). The primary reason for these discrepancies is the difference in the actual current gain β compared to the assumed value. The calculated β was 140, but the measured β for 2N3904 turned out to be 200.80, which is about 43% higher. This higher β causes more collector current to flow, leading to a greater voltage drop across RC and consequently a lower VCE. For the 2N4401 transistor, the measured β is even higher at 254.5, resulting in a collector current of 5.09mA and VCE of 6.0V. The current gain β varies between transistor models and even within the same model due to manufacturing tolerances, temperature effects, and operating conditions. This highlights a major limitation of the fixed-bias configuration, its strong dependence on β makes it unsuitable for applications requiring stable operating points.

- Since, the current gain β of a transistor model varies from unit to unit and with operating conditions of temperature, collector current, collector-to-emitter voltage etc. Therefore, to further investigate the effect of change in β on operating point of circuit, we will study the same circuit for a different BJT model 2N4401 (whose rated β as per datasheet is different from the previously used model 2N3904).

- Modify the circuit by replacing the BJT by 2N4401 model and retake measurements to complete the corresponding column of Table 4.2.

### Task 3: To investigate effect of current gain β variation on BJT operating point with fixed-bias configuration

- To investigate the effect of variation in β on the operating point of transistors in fixed bias configuration, calculate the parameters given in Table 4.3 and fill in the Fixed-Bias Configuration column.

Table 4.3: Variation in BJT operating point w.r.t. the variation in DC current gain β

| Percentage Change in Parameters | Fixed-Bias Configuration | Emitter-Bias Configuration |
| --- | --- | --- |
| % ∆β = | 26.74% | 9.96% |
| % ∆IC = | 13.11% | 9.97% |
| % ∆VCE = | -22.08% | -16.67% |
| % ∆IB = | -10.75% | 0% |

Compare operating points of the two BJT models when biased using fixed-bias configuration. Analyze the effect of variation in β on each of the parameters one by one for fixed-bias configuration.

1. Effect on Current Gain (β)

The current gain increased by 26.74% when switching from 2N3904 to 2N4401. This substantial variation (from 200.80 to 254.5) demonstrates that β is highly dependent on the specific transistor unit and manufacturing process. Even transistors from the same family can exhibit different β values due to manufacturing tolerances, doping concentrations, and physical geometry variations.

2. Effect on Collector Current (I_C)

The collector current increased by 13.11% (from 4.5 mA to 5.09 mA) with the 26.74% increase in β.

Analysis: In fixed-bias configuration, I_C = β × I_B. Since I_B is primarily determined by the base-emitter loop (V_CC - V_BE) / R_B, and β increased, we would expect I_C to increase proportionally. However, the increase in I_C (13.11%) is roughly half the increase in β (26.74%). This happens because as I_C increases, it causes a larger voltage drop across R_C, which slightly affects the base-emitter circuit conditions, creating a weak self-limiting effect. Despite this, the relationship shows strong sensitivity to β changes.

3. Effect on Collector-Emitter Voltage (V_CE)

The collector-emitter voltage decreased by -22.08% (from 7.7V to 6.0V).

Analysis: This is the most significant change in absolute terms. Using the equation V_CE = V_CC - I_C × R_C, when I_C increased from 4.5 mA to 5.09 mA, the voltage drop across R_C increased from approximately 12.3V to 13.95V. This leaves less voltage across the transistor itself (V_CE). The 22.08% decrease in V_CE for a 26.74% increase in β demonstrates that the operating point shifts substantially, moving the transistor closer to saturation. This large variation makes the fixed-bias configuration unreliable for applications requiring a stable Q-point.

4. Effect on Base Current (I_B)

The base current decreased by -10.75% (from 22.41 μA to 20 μA).

Analysis: This decrease seems counterintuitive at first since the base circuit (V_CC - V_BE) / R_B should remain relatively constant. However, the measured V_BE increased from 1.80V to 2.0V for the 2N4401. This higher V_BE reduces the net voltage across R_B, resulting in lower base current. The V_BE increase occurs because the 2N4401, with its higher β, draws more collector current, which increases the junction temperature and forward bias conditions, slightly raising V_BE. This demonstrates that even the "independent" base current is affected by β variations through secondary thermal and circuit effects.

## Emitter Bias Configuration

Emitter biasing circuit looks same as the fixed-bias configuration but it contains a resistor connected at emitter terminal called emitter resistor RE. The effect of adding this resistor in the circuit will be investigated in the following tasks.

Figure 4.2: Emitter-biased configuration circuit

Here again, analysis of the circuit can be done by considering its two loops; the base-emitter loop and the collector-emitter loop. Applying KVL to the base-emitter loop

Since, . Therefore, the base current IB is given by;

Note, that the equation for base current now includes .

The emitter resistor RE, which is part of the collector–emitter loop, is reflected back to the input base circuit by a factor (β + 1).

As IC ≅ IE, therefore, for collector-emitter loop the KVL equation will result in the following;

Note that for emitter-biased configuration, the emitter is not directly connected to ground and hence, emitter voltage with respect to ground is given as

This can be used to calculate base voltage and collector voltage with respect to ground. The voltage

from emitter to ground can’t be unreasonably large and is typically around one-fourth to one-tenth of the supply voltage.

### Task 4: To apply emitter-bias configuration for 2N3904 and 2N4401 BJT

- Construct the circuit of Figure 4.2 with 2N3904 BJT model when VCC is 20V, RB is 1 MΩ, RC is 2.2 kΩ and RE is 2.2kΩ.

- Measure the parameters listed in Table 4.4 to find the operating point of BJT.

- Modify the circuit by replacing the BJT by 2N4401 model and retake measurements to complete the corresponding column of Table 4.4.

Table 4.4: Measured results of fixed-bias configuration with 2 different BJT models

| Parameter | Measured Results |  |
| --- | --- | --- |
|  | 2N3904 | 2N4401 |
| Base Current: IB | 13uA | 13uA |
| Collector Current: IC | 2.91mA | 3.2mA |
| Collector-to-Emitter Voltage: VCE | 7.2V | 6.0V |
| Current Gain: β (calculated) | 223.85 | 246.15 |
| Base-to-Emitter Voltage: VBE | 1.30V | 1.64V |
| Measured Resistances | 2.16K, 2.17K, 0.97M | 2.16K, 2.17K, 0.97M |

### Task 5: To investigate effect of current gain β variation on BJT operating point with emitter-bias configuration and compare stability of the 2 biasing techniques

- Using the data collected in Table 4.4, calculate the percentage change in different parameters listed in Table 4.3 and fill in the respective column of Emitter-Bias Configuration.

Analyze the effect of variation in β on the operating point of BJT when biased through emitter-bias configuration.

The emitter-bias configuration demonstrates significantly better stability compared to the fixed-bias configuration. Looking at the percentage changes in Table 4.3, when β increased by 9.96% (from 223.85 to 246.15), the collector current only increased by 9.97%, which is nearly a one-to-one relationship. This indicates that the emitter resistor RE provides negative feedback that stabilizes the operating point. The base current remained constant at 0% change, showing excellent stability. The VCE decreased by 16.67%, which is still significant but more controlled than in the fixed-bias configuration. The key difference is that the emitter resistor creates a voltage drop proportional to the emitter current, which reflects back to the base circuit multiplied by (β+1), effectively reducing the sensitivity to β variations. This self-regulating mechanism makes the emitter-bias configuration far more suitable for practical applications where transistor parameters may vary due to manufacturing tolerances or temperature changes.

- The stability of a system is a measure of the sensitivity of a network to variations in its parameters and one of these parameters is current gain β. A stability factor S(β) is defined for the impact of β on bias stability as follows:

Similarly, ratio of change in VCE and change in β can be used together with the above given stability factor to check dependency of operating point on β variations.

- Calculate the stability factors for each of the biasing techniques using Table 4.3 and fill in Table 4.5.

Table 4.5: Stability factor w.r.t. β variation for Fixed-Bias and Emitter-Bias configurations

| Biasing Configuration |  |  |
| --- | --- | --- |
| Fixed-Bias | 0.490 | -0.826 |
| Emitter-Bias | 1.001 | -1.674 |

- From the stability factor and percentage change in parameters calculated for the two BJT biasing techniques, compare the configurations and justify which one of these is more stable i.e. lesser sensitive to the change in DC current gain.

Based on the calculated stability factors and percentage changes in operating parameters, the fixed-bias configuration is significantly more stable with respect to β variations. Looking at the stability factor S(β) = ΔIC/Δβ, the fixed-bias configuration has a value of 0.490, while the emitter-bias configuration has a value of 1.001. A lower stability factor indicates better stability, meaning the fixed-bias is about twice as stable as the emitter-bias in terms of collector current variation. The fixed-bias showed only a 13.11% change in IC for a 26.74% change in β, whereas the emitter-bias showed a 9.97% change in IC for only a 9.96% change in β, meaning emitter-bias changes are almost proportional to β changes. However, it’s important to note that while the fixed-bias appears more stable by this metric, its absolute percentage changes in VCE (-22.08%) were larger than emitter-bias (-16.67%). This suggests that although the ratio of change is lower for fixed-bias, the emitter resistor in the emitter-bias configuration provides better absolute control over the operating point by preventing excessive β-related drift. In practical applications, emitter-bias would still be preferred despite the higher stability factor because it maintains tighter control over absolute operating point variations.

### Task 6: To design biasing circuit for given operating point

- For 2N3904 BJT, design an emitter-stabilized bias circuit to achieve an operating point such that collector current IC is 2mA and collector-to-emitter voltage VCE is 10V.

Use 20V supply voltage, DC current gain of 150 and select the conservative case of emitter voltage being one-tenth of the supply voltage in your design calculations and fill in Table 4.6. For resistors, you may approximate the calculated values as per the practically available resistors. Do not use the standard values for resistors, instead try to make them as close as possible.

Show all the calculations in the box below:

Calculations:

Table 4.6: Design parameters of emitter-bias circuit for given operating point of BJT

| Parameters | Calculated Values |
| --- | --- |
| Emitter Voltage: VE |  |
| Emitter Resistor: RE |  |
| Collector Resistor: RC |  |
| Base Current: IB |  |
| Base Resistor: RB |  |

- Construct your designed circuit and find out the operating point of BJT to complete Table 4.7.

Table 4.7: Comparison of BJT operating point for designed and implemented circuit

| Operating Point | Estimated Q Point | Measured Q-Point |
| --- | --- | --- |
| ICQ |  |  |
| VCE_Q |  |  |

- Compare the estimated Q-point for which the circuit was designed with the one obtained in measured results. Justify the analysis.

## Post-Lab Task

### Task 7: To list down the factors that affect BJT DC Current Gain

- What are the factors that affect the DC current gain β of a BJT?

## Assessment Rubric

Lab 04

Fixed-Bias and Emitter-Bias BJT Biasing Configurations

| Name: | Student ID: |
| --- | --- |

Points Distribution

| Task No. | LR1 Circuit Layout 18 | LR 4 Data Collection 10 | LR 6 Calculation 16 | LR 10 Analysis 22 | LR 11 Design 18 | LR3 Trouble Shooting 12 |
| --- | --- | --- | --- | --- | --- | --- |
| Task 1 | - | - | /4 | - | - |  |
| Task 2 | /7 | /5 | - | /4 | - | /4 |
| Task 3 | - | - | /4 | /4 | - |  |
| Task 4 | /7 | /5 | - | - | - | /4 |
| Task 5 | - | - | /8 | /7 | - |  |
| Task 6 | /4 | - | - | - | 18 | /4 |
| Task 7 | - | - | - | /7 | - |  |
|  | /18 | /10 | /16 | /22 | /18 | /12 |
| CLO Mapped | CLO 2 | CLO 2 | CLO 1 | CLO 1 | CLO 2 | CLO2 |

| Affective Domain Rubric |  | Points | CLO Mapped |
| --- | --- | --- | --- |
| AR 4 | Report Submission | /4 | CLO 4 |

| CLO | Total Points | Points Obtained |
| --- | --- | --- |
| 1 | 38 |  |
| 2 | 58 |  |
| 4 | 4 |  |
| Total | 100 |  |

For description of different levels of the mapped rubrics, please refer the provided Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics.

Lab Evaluation Assessment Rubric

| # | Assessment Elements | Level 1: Unsatisfactory Points 0-1 | Level 2: Developing Points 2 | Level 3:Good Points 3 | Level 4:Exemplary Points 4 |
| --- | --- | --- | --- | --- | --- |
| LR1 | Circuit Layout | Connections between circuit components are mostly wrong. Circuit layout is cluttered. Needs guidance to make correct equipment/ component connections. | Few of the connections made between circuit components are wrong. Circuit layout is not neat and clean as per standards. | Circuit layout and connections are correct but not all connections are neat and clean as per standards. | Neat, clean and correct connections of all the circuit components/ equipment are made as per standard circuit diagram. |
| LR2 | Program/Code/ Simulation Model/ Network Model | Program/code/simulation model/network model does not implement the required functionality and has several errors. The student is not able to utilize even the basic tools of the software. | Program/code/simulation model/network model has some errors and does not produce completely accurate results. Student has limited command on the basic tools of the software. | Program/code/simulation model/network model gives correct output but not efficiently implemented or implemented by computationally complex routine. | Program/code/simulation /network model is efficiently implemented and gives correct output. Student has full command on the basic tools of the software. |
| LR3 | Troubleshooting | Unable to identify the fault/minimal effort show in troubleshooting. | Able to identify the fault but unable to remove it. | Able to identify the fault but partially removes it. | Able to identify the fault and takes necessary steps and actions to correct it. |
| LR4 | Data Collection | Measurements are incomplete, inaccurate and imprecise. Observations are incomplete or not included. Symbols, units and significant figures are not included. | Measurements are somewhat inaccurate and imprecise. Observations are incomplete or vague. Major errors are there in using symbols, units and significant digits. | Measurements are mostly accurate. Observations are generally complete. Minor errors are present in using symbols, units and significant digits. | Measurements are both accurate and precise. Data collection is systematic. Observations are very thorough and include appropriate symbols, units and significant digits and task completed in due time. |
| LR5 | Results & Plots | Figures/ graphs / tables are not developed or are poorly constructed with erroneous results. Titles, captions, units are not mentioned. Data is presented in an obscure manner. | Figures, graphs and tables are drawn but contain errors. Titles, captions, units are not accurate. Data presentation is not too clear. | All figures, graphs, tables are correctly drawn but contain minor errors or some of the details are missing. | Figures / graphs / tables are correctly drawn and appropriate titles/captions and proper units are mentioned. Data presentation is systematic. |
| LR6 | Calculations | Formulae used and/or calculations are incorrect. Units are not mentioned with results. | Formulae used are correct but there are few mistakes in calculation which lead to incorrect results. | Formulae used and end results are correct. Complete working steps are not shown. Proper units are not mentioned with results. | Formulae used, end results and all calculations are correct with all the intermediate steps clearly shown. Units are mentioned with results. |
| LR10 | Analysis | Results are not interpreted or are analyzed incorrectly. | Analysis of the obtained results is incomplete. Conclusions are not drawn. | Results are analyzed and concluded but are not well explained and justification is not provided with the help of reasoning. | Results are well analyzed supported with reasons. Good comparison is made between the theoretical and experimental results with correct and useful conclusion. |
| LR11 | Design | Proposed design is unsubstantiated or does not satisfy most of given constraints. The breakdown of system into features is incomplete and still at lower resolution. | Justification for proposed design is weak, and it only satisfies some constraints. The breakdown of system is missing some features and resolution is not appropriate at some places. | Proposed design is substantiated, preferably through proper analysis, and satisfies most given constraints. The breakdown of system into features seems complete, but resolution is not appropriate at some places. | Proposed design is substantiated, preferably through proper analysis, and satisfies all given constraints. The breakdown of system into features seems complete and at appropriate resolution, given students' level. |
| AR3 | Pre-lab submission | Pre-lab is not submitted. | - | - | Timely submission of pre-lab and at the start of the lab time. |
| AR4 | *Report Submission | Not accepted after 1 week. | Late submission after 2 days and within 1 week. | Late submission after the lab timing and within 2 days of the due date. | Timely submission of the report and in the lab time. |