# Lab 05

# Collector Feedback Bias and Voltage Divider Bias BJT Biasing Configurations

| Name: Syed Sualeh Abbas, Muhammad Haider | Student ID: 10303 |
| --- | --- |

- Objective

To investigate the effect of current gain β variation on quiescent operating conditions of a Bipolar Junction Transistor in collector feedback bias and voltage divider bias configurations.

Note: Pre-lab (printed or hand-written) should be done on a separate paper (use A4 sheet) and will be submitted at the start of Lab within first 5 minutes. All the plots to be attached, should be labeled properly, followed by Task number and must be readable, i.e., use white background, suitable marker color (recommended – any dark color) and increase the width of trace. On PSpice there is an option to add text labels in the plots.

- Introduction

In previous lab, the stability of fixed-bias and emitter-bias BJT biasing configurations have been investigated. In this lab, you will investigate the dependency of BJT operating point on DC current gain β for two more circuits of BJT biasing; collector feedback bias (with and without emitter resistor) and voltage divider bias configuration.

## Pre-Lab Task

- Collector Feedback Bias Configuration Without Emitter Resistor

Figure 1 shows BJT biasing through collector-feedback bias configuration. In this circuit, there is a feedback path from collector to base through a feedback resistor represented as RF. The emitter terminal is directly grounded.

Figure 1: Collector-Feedback Bias configuration without emitter resistor

It is important to note here that the current through RC is not IC, but IC’ (where, IC’= IC + IB). Since, base current is small as compared to collector current, therefore, the approximation (IC’≅ IC) is normally applied. The analysis of the circuit can be performed by writing KVL equations for base-emitter and collector emitter loop.

The feedback path results in a reflection of the resistance RC back to the input circuit, much like the reflection of RE in emitter-bias circuit. Similarly, for collector-emitter loop, KVL equation gives;

Task 2: To bias 2N3904 and 2N4401 BJT models using Collector Feedback Bias configuration circuit without RE

- Implement the circuit shown in Figure 1 to bias 2N3904 BJT with VCC=20V, RC = 3kΩ and RF= 390kΩ.

- Measure the parameters listed in Table 1.

Table 1: Results of BJT biasing with collector-feedback biasing circuit without emitter resistor

| Parameter | Results |  |
| --- | --- | --- |
|  | 2N3904 | 2N4401 |
| Base Current: IB | 32.5uA | 29.4uA |
| Collector Current: IC | 4.32mA | 4.25mA |
| Collector-to-Emitter Voltage: VCE | 7.19V | 7.4V |
| Current Gain: β (calculated) | 132.92 | 144.55 |
| Base-to-Emitter Voltage: VBE | 0.668V | 0.64V |
| Measured resistances | 2.93K, 13.2M | 2.93K, 13.2M |

- Now, replace the BJT model 2N3904 by 2N4401 model in the same circuit and take measurements to complete Table 1.

### Task 3: To investigate effect of current gain β variation on BJT operating point with collector feedback-bias configuration circuit without RE

- When biased using collector feedback-bias circuit, analyze the variation in operating point of BJTs as the DC current gain varies from the results of Table 1.

- Calculate the percentage change in parameters listed in Table 4 using the results of Task 1.

- Collector Feedback Bias Configuration with Emitter Resistor

In the collector-feedback biasing configuration of Figure 1, a resistor at emitter RE is added as shown in Figure 2, that modifies the equations for base current and collector-emitter voltage as follows;

Figure 2: Collector-feedback circuit with emitter resistor

### Task 4: To bias 2N3904 and 2N4401 BJT models using Collector Feedback Bias configuration circuit with RE

- Modify the implemented circuit to include emitter resistor RE of 2.2 kΩ. Again, take the measurements for the specified BJT models (2N3904 and 2N4401) and note down in Table 2.

Table 2: Results of BJT biasing with collector-feedback biasing circuit having emitter resistor

| Parameter | Results |  |
| --- | --- | --- |
|  | 2N3904 | 2N4401 |
| Base Current: IB | 17.54uA | 17.23uA |
| Collector Current: IC | 2.86mA | 2.86mA |
| Collector-to-Emitter Voltage: VCE | 5.2V | 5.7V |
| Current Gain: β (calculated) | 163.05 | 165.98 |
| Base-to-Emitter Voltage: VBE | 0.66V | 0.65V |
| Measured Resistance | 2.93K, 13.2M, 2.1K | 2.93K, 13.2M, 2.1K |

### Task 5: To investigate effect of current gain β variation on BJT operating point with collector feedback-bias configuration circuit with RE

- After addition of emitter resistor, analyze the variation in operating point of BJTs as the DC current gain varies from the results of Table 2. Compare it with the case when emitter was directly grounded. Using the expressions of base-current for each case, relate the effect of emitter resistor on the BJT operating point.

- Calculate the percentage change in parameters listed in Table 4 using the results of Task 3.

- Voltage-Divider Bias Configuration

The circuit shown in Figure 3 is called Voltage-Divider bias configuration as a voltage divider circuit using resistors R1 and R2 is implemented to establish base voltage. There are two approaches to analyze the circuit, the exact and the approximate one. Here, approximate one is used for analysis.

Figure 3: Voltage-Divider Bias Configuration

For the circuit to operate efficiently, it is assumed that the current through R1 and R2 should be approximately equal to and much larger than the base current (at least 10:1). If the biasing circuit fulfills the condition, then

Base voltage can be then expressed as:

Similarly, the collector-to-emitter voltage can be expressed as:

### Task 6: To bias 2N3904 and 2N4401 BJT models using Voltage Divider Bias configuration circuit

- Implement the circuit shown in Figure 3 to bias 2N3904 BJT model with VCC= 20V, R1= 33kΩ, R2= 6.8kΩ , RC = 1.8kΩ and RE= 680 Ω.

- Take the measurements for parameters listed in Table 3.

- Now, replace 2N3904 BJT model by 2N4401 and complete the measurements for Table 3.

Table 3: Results of BJT biasing through voltage-divider bias configuration

| Parameter | Results |  |
| --- | --- | --- |
|  | 2N3904 | 2N4401 |
| Base Current: IB | 14.47uA | 15uA |
| Collector Current: IC | 3.9mA | 3.94mA |
| Collector-to-Emitter Voltage: VCE | 10.19V | 10.1V |
| Current Gain: β (calculated) | 269.52 | 262.66 |
| Base-to-Emitter Voltage: VBE | 0.66V | 0.64V |
| Measured Resistances | 675, 1.82K, 7.2K, 39.4K | 675, 1.82K, 7.2K, 39.4K |

### Task 7: To investigate effect of current gain β variation on BJT operating point with Voltage-Divider Bias configuration circuit

- Using the results of Table 3, analyze the effect of DC current gain variation on the operating point of BJT when it is biased using voltage-divided bias circuit.

- Calculate the percentage change in parameters listed in Table 4 using the results of Task 5.

Table 4: Variation in operating point compared with change in DC current gain for different biasing circuits

| Percentage Change in Parameters | Collector Feedback Bias without RE | Collector Feedback Bias with RE | Voltage-Divider Bias |
| --- | --- | --- | --- |
| % ∆β = | [(144.55 - 132.92)/132.92] × 100 = 8.75% | [(165.98 - 163.05)/163.05] × 100 = 1.80% | [(262.66 - 269.52)/269.52] × 100 = -2.54% |
| % ∆IC = | [(4.25 - 4.32)/4.32] × 100 = -1.62% | [(2.86 - 2.86)/2.86] × 100 = 0% | [(3.94 - 3.9)/3.9] × 100 = 1.03% |
| % ∆VCE = | [(7.4 - 7.19)/7.19] × 100 = 2.92% | [(5.7 - 5.2)/5.2] × 100 = 9.62% | [(10.1 - 10.19)/10.19] × 100 = -0.88% |
| % ∆IB = | [(29.4 - 32.5)/32.5] × 100 = -9.54% | [(17.23 - 17.54)/17.54] × 100 = -1.77% | [(15 - 14.47)/14.47] × 100 = 3.66% |

### Task 8: To compare the stability of BJT biasing configurations

- Using results of Table 4, compute the stability factors for each of the implemented biasing techniques. Note down the results of fixed-bias and emitter-bias circuits from previous lab to complete Table 5.

Table 5: Stability factors of different BJT biasing techniques w.r.t. the variation in DC current gain

| Biasing Configuration |  |  |
| --- | --- | --- |
| Fixed-Bias (Lab 04) | 0.49 | -0.825 |
| Emitter-Bias (Lab 04) | 1.001 | -1.674 |
| Collector-Feedback Bias without RE |  |  |
| Collector-Feedback Bias with RE |  |  |
| Voltage-Divider Bias |  |  |

- Compare the performance of BJT biasing techniques using the above calculated performance parameters. What does the magnitude of calculated ratio signify? Which of the 5 investigated circuits can be regarded as β independent circuit?

- Post-Lab Task

### Task 9: To design voltage-divider BJT biasing circuit for given operating point

Design a voltage-divider BJT biasing circuit to make BJT 2N3904 operate at the operating point given in Lab 04 (Task 6) where, the required collector current IC was 2mA and collector-to-emitter voltage VCE was 10V.

Use 20V supply voltage, DC current gain of 150 and select the conservative case of emitter voltage being one-tenth of the supply voltage in your design calculations. For design, you may use approximate analysis for which equations are given in section 5.6. Show all the calculations.

Simulate your designed circuit.

Assessment Rubric

Lab 05

Collector Feedback Bias and Voltage Divider Bias BJT Biasing Configurations

| Name: | Student ID: |
| --- | --- |

Points Distribution

| Task No. | LR1 Circuit | LR2 Simulation | LR 4 Data Collection | LR 6 Calculation | LR 10 Analysis | LR 11 Design |
| --- | --- | --- | --- | --- | --- | --- |
| Task 1 |  |  | /10 |  |  |  |
| Task 2 | /4 |  | /4 | /2 |  |  |
| Task 3 |  |  |  | /4 | /4 |  |
| Task 4 | /4 |  | /4 | /2 |  |  |
| Task 5 |  |  |  | /4 | /4 |  |
| Task 6 | /4 |  | /4 | /2 |  |  |
| Task 7 |  |  |  | /4 | /4 |  |
| Task 8 |  |  |  | /4 | /8 |  |
| Task 9 |  | /6 |  |  |  | /12 |
|  | /12 | /6 | /22 | /22 | /20 | /12 |
| CLO Mapped | CLO 1 | CLO1 | CLO 1 | CLO 1 | CLO 1 | CLO 2 |

| Affective Domain Rubric |  | Points | CLO Mapped |
| --- | --- | --- | --- |
| AR4 | Pre-Lab Submission | /2 | CLO 4 |
| AR 4 | Report Submission | /4 | CLO 4 |

| CLO | Total Points | Points Obtained |
| --- | --- | --- |
| 1 | 82 |  |
| 2 | 12 |  |
| 4 | 06 |  |
| Total | 100 |  |

For description of different levels of the mapped rubrics, please refer the provided Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics.