# Lab 09

# MOSFET Biasing

| Name: | Student ID: |
| --- | --- |

## Objective

To investigate the effect of variation in threshold voltage of enhancement type n-channel MOSFET on its operating point in voltage-divider bias and self-bias configurations

## Introduction to MOSFET Biasing

In previous lab, three different regions of operation; cut-off region, triode region and saturation region, have been identified in characteristic curve of MOSFET. From the identified regions, it can be concluded that MOSFET acts like a voltage-controlled resistor in triode region where the drain current is dependent on the drain to source voltage and can be given as;

In saturation region, the MOSFET operates as a voltage-controlled current source with the control relationship described by

Note that in the above equation, drain current is dependent on the gate-to-source voltage and effect of channel length modulation is not considered which makes it independent of drain-to-source voltage as shown in the characteristic curve of Figure 1.

Figure 1: MOSFET drain characteristic curve

In the expressions above, the first factor; process trans-conductance parameter Kn’, is determined by the process technology used to fabricate the MOSFET. It is the product of the electron mobility, μn, and the oxide capacitance, Cox. The second term is transistor aspect ratio (W/L) with channel width W and length L. Difference between gate-to-source voltage and threshold voltage is called over-drive voltage Vov.

An appropriate DC operating point or bias point is characterized by a stable and predictable drain current ID and by a drain-to-source voltage VDS. This is stablished through biasing configurations and for operation of MOSFET as amplifier, the biasing should ensure operation in saturation region.

The value of the threshold voltage Vt, the oxide-capacitance Cox, and (to a lesser extent) the transistor aspect ratio varies widely among devices of supposedly the same size and type. This is certainly the case for discrete devices, in which large spreads in the values of these parameters occur among devices of the same manufacturer’s part number. The spread is also large in integrated circuits, especially among devices fabricated on different wafers and certainly between different batches of wafers. Furthermore, both Vt and μn depend on temperature. A biasing technique that makes the drain current least dependent on these variations is considered to be stable.

In this lab, we will be investigating the following different biasing configurations with respect to the change in threshold voltage Vt:

- Biasing by fixing gate-to-source voltage VGS

- Biasing by fixing gate-to-source voltage VGS with degeneration resistance

Biasing by connecting gate-to-drain feedback resistor (or, self-bias configuration) is categorized as enhancement type and depletion type.

## 9.3. Pre-Lab Task

## Task 1:

- What is Channel Length Modulation? Explain its impact on MOSFET parameters.

## Define MOSFET Gate Capacitance.

### Task 2: To create PSpice models of NMOS with given specifications

Before simulating different biasing techniques, we will create two different models of NMOS in PSpice with given specifications of threshold voltage, process trans-conductance parameter and aspect ratio. To do this, we will be using MbreakN PSpice model.

- Add an MbreakN3 PSpice model in your design. You can add this from place part option by searching MbreakN3 or directly place it from Place> PSpice Component> Discrete> NMOS.

- Right click on the added part and select Edit PSpice Model option. In model editor, enter the following parameters using the labels described below:

- Model Name: MOS1

- Part Type: NMOS

- Threshold Voltage: VTO =1V

- Process Trans-conductance: KP=0.1mA/V2 (This is same as Kn’= μnCox)

- Width: W=2.5um

- Length: L=0.25um

For your reference, capture of model editor window is shown in Figure 2. This will add an NMOS model with specifications mentioned above.

Figure 2: Capture of model editor window for NMOS1 parameters

- Add another NMOS model named MOS2 with threshold voltage of 1.5V keeping all the other parameters constant.

- You have successfully modeled two NMOS models in PSPice with slightly different threshold voltages. Verify the characteristics by obtaining ID-VGS curves of these MOSFETs by keeping drain-to-source voltage Vdd constant (10V) and sweeping gate-to-source Vgs voltage from 0 to 3V in steps of 0.1V. Add captures of the obtained graphs (with white background). You can obtain both the plots in one window through one simulation by connecting both MOSFETs with same Vdd and Vgs DC sources. Reference circuit is shown in Figure 3.

Figure 3: Schematic for obtaining ID-VGS curves of modelled MOSFETs

## 9.4. Biasing by Fixing VGS or Voltage-Divider Biasing Configuration

The most straightforward approach to biasing a MOSFET is to fix its gate-to-source voltage VGS to the value required to provide the desired ID. This voltage value can be derived from the power-supply voltage VDD through the use of an appropriate voltage divider. Figure 4 shows voltage-divider biasing configuration. Here, resistors R1 and R2 create voltage-divider to establish VG and RD will determine the drain voltage as given below.

Initially, for calculations it is assumed that the MOSFET will operate in saturation region, therefore, calculations can be done using the equations for saturation region and then it is verified that the condition of voltages for saturation region is valid for the obtained results. Note that in this configuration source is directly grounded.

Figure 4: Voltage-divider bias configuration

### Task 3: To bias the created n-channel MOSFET models in voltage-divider bias configuration

- Bias the created MOSFET models (NMOS1 and NMOS2) using voltage-divider bias configuration with VDD=15 V, R1= 8mega Ω, R2=1.23mega Ω and RD=10kΩ.

- Create a new simulation and obtain the values listed in Table 1.

Table 1: Simulated results of voltage-divider bias configuration

| Measurements | NMOS 1 | NMOS 2 |
| --- | --- | --- |
| Threshold Voltage: Vt |  |  |
| Drain Current: ID |  |  |
| Drain-to-Source Voltage: VDS |  |  |
| Gate-to-Source Voltage: VGS |  |  |
| Mode of Operation |  |  |

## 9.5. Biasing by Fixing VG and Connecting a Resistance in the Source

It consists of fixing the voltage at the gate, VG, and connecting a resistance in the source lead as shown in Figure 5. Here, resistor RS provides negative feedback, which acts to stabilize the value of the bias current ID. When ID increases for whatever reason, since VG is constant, VGS will have to decrease as drop across RS will increase the source voltage. This in turn results in a decrease in ID. Thus the action of RS works to keep ID as constant as possible. This negative feedback action of RS gives it the name degeneration resistance.

Figure 5: Voltage-divider bias configuration with source degeneration resistance

### Task 4: To bias the created n-channel MOSFET models in voltage-divider bias configuration with source degeneration resistance

- Bias the created MOSFET models (NMOS1 and NMOS2) using voltage-divider bias configuration with source resistance shown in Figure 5. Set VDD=15 V, R1= 8mega Ω, R2=7mega Ω, RD=10kΩ and RS=10kΩ.

- Simulate the circuit and obtain the values listed in Table 2.

Table 2: Simulated results of voltage-divider with source resistance bias configuration

| Measurements | NMOS 1 | NMOS 2 |
| --- | --- | --- |
| Threshold Voltage: Vt |  |  |
| Drain Current: ID |  |  |
| Drain-to-Source Voltage: VDS |  |  |
| Gate-to-Source Voltage: VGS |  |  |
| Mode of Operation |  |  |

## 9.6. Biasing by Drain-to-Gate Feedback Resistor (Self-Bias Configuration)

A simple and effective discrete-circuit biasing arrangement utilizing a feedback resistor connected between the drain and the gate is shown in Figure 6. Here, the large feedback resistance RG (usually in the mega ohm range) forces the voltage at the gate to be equal to that at the drain (because IG = 0). If ID for some reason increases, then VDS (and VGS) decreases. The decrease in VGS in turn causes a decrease in ID, a change that is opposite in direction to the one originally assumed. Thus the negative feedback or degeneration provided by RG works to keep the value of ID as constant as possible.

Figure 6: Biasing circuit with

### Task 5: To bias the created n-channel MOSFET models using self-bias configuration

- Bias the created MOSFET models (NMOS1 and NMOS2) using the self-bias circuit shown in Figure 6. Set VDD=15 V, RD=10kΩ, RS=10kΩ and RG=10mega Ω.

- Simulate the circuit and obtain the values listed in Table 3.

Table 3: Simulated results of voltage-divider bias configuration

| Measurements | NMOS 1 | NMOS 2 |
| --- | --- | --- |
| Threshold Voltage: Vt |  |  |
| Drain Current: ID |  |  |
| Drain-to-Source Voltage: VDS |  |  |
| Gate-to-Source Voltage: VGS |  |  |
| Mode of Operation |  |  |

### Task 6: To investigate the performance of simulated biasing techniques with respect to change in threshold voltage

To investigate and compare the performance of implemented biasing circuits, calculate the percentage change in the parameters listed in Table 4 using the data collected in Table 1, 2 and 3.

Table 4: Effect of change in threshold voltage on drain current for different biasing techniques

| Percentage Change in Parameters | Fixed VGS | Fixed VG with RS | Self-Bias |
| --- | --- | --- | --- |
| % ∆ID = |  |  |  |
| % ∆VT = |  |  |  |
| Stability Factor: |  |  |  |

### Task 7: To identify the effect of load resistor on MOSFET mode of operation in different biasing configurations

- Simulate the biasing circuits of Figure 5 and Figure 6 with RD = 30kΩ for NMOS1 model. Take measurements to fill-in Table 5. Identify mode of operation using results.

Table 5: Biasing of NMOS1 at 30kOhms RD by voltage-divider bias with Rs and self-bias circuits

| Measurements | Voltage-Divider Bias with Rs | Self-Bias |
| --- | --- | --- |
| Threshold Voltage: Vt |  |  |
| Drain Current: ID |  |  |
| Drain-to-Source Voltage: VDS |  |  |
| Gate-to-Source Voltage: VGS |  |  |
| Mode of Operation |  |  |

## 9.7. Post Lab Task

### Task 8: To analyze the performance of implemented biasing techniques

- Analyze the results obtained in Task 6 and Task 7 as demonstrated below:

- For Task 6, analyze the effect of variation in threshold voltage on drain current and try to justify the effect for each biasing technique in terms of the circuit components/ equations / measurements (e.g. values of VGS collected in Tables).

- For Task 7, justify the mode of operation with change in drain resistance for the two circuits. Why mode of operation is different for each?

## Assessment Rubric

Lab 09

MOSFET Biasing

| Name: | Student ID: |
| --- | --- |

Points Distribution

| Task No. | LR1/2 Simulation 40 | LR 4 Data Collection 32 | LR 5 Plots 4 | LR 6 Calculation 4 | LR 10 Analysis 16 |
| --- | --- | --- | --- | --- | --- |
| Task 1 | - | - | - | - | /4 |
| Task 2 | /8 | - | /4 | - | - |
| Task 3 | /8 | /8 | - | - | - |
| Task 4 | /8 | /8 | - | - | - |
| Task 5 | /8 | /8 | - | - | - |
| Task 6 | - | - | - | /4 | - |
| Task 7 | /8 | /8 | - | - | - |
| Task 8 | - |  | - | - | /12 |
| CLO Mapped | CLO 3 | CLO 1 | CLO 3 | CLO 1 | CLO 1 |
|  | /40 | /32 | /4 | /4 | /16 |

| Affective Domain Rubric |  | Points | CLO Mapped |
| --- | --- | --- | --- |
| AR 4 | Report Submission | /4 | CLO 4 |

| CLO | Total Points | Points Obtained |
| --- | --- | --- |
| 1 | 52 |  |
| 3 | 44 |  |
| 4 | 04 |  |
| Total | 100 |  |

For description of different levels of the mapped rubrics, please refer the provided Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics.