Syed Sualeh Abbas 10303

Muhammad Hiader 10270

## Objective

To investigate the characteristics of MOSFET (Metal Oxide Semiconductor Field Effect Transistor) through its characteristic curves

## Introduction to MOSFETs

MOSFETs (Metal-Oxide-Semiconductor Field-Effect Transistors) are the type of FETs (field-effect transistors) in which the metal gate electrode is separated from the semiconductor by an insulator and therefore, also known as Insulated Gate Field Effect Transistors (IGFET). Unlike the current-controlled bipolar device (BJT), which requires base current to allow the current to flow in the collector, the MOSFET device is a voltage-controlled unipolar device because the conduction is controlled by voltage and it uses only the majority carriers in conduction. MOSFETs surpass all other devices in switching speed, where switching speeds can exceed a few megahertz. MOSFETs can be mainly categorized as enhancement type and depletion type.

## Pre-Lab Task

## Task 1:

- Mention various types of MOSFET and applications of MOS.

## 1. Types of MOSFET ## MOSFETs are mainly classified based on their mode of operation and type of charge carrier. ## A. Based on Mode of Operation ## Enhancement Mode MOSFET (E-MOSFET) ## The transistor is normally OFF when VGS=0. ## A gate voltage must be applied to create a conduction channel between drain and source. ## Widely used in digital circuits and switching applications. ## Depletion Mode MOSFET (D-MOSFET) ## The transistor is normally ON when VGS=0. ## Applying a gate voltage can reduce (deplete) the channel current. ## Used in analog circuits and amplifiers. ## B. Based on Channel Type ## N-Channel MOSFET ## Uses electrons as majority carriers. ## Has higher mobility, therefore faster switching speed. ## Commonly used in high-speed switching circuits. ## P-Channel MOSFET ## Uses holes as majority carriers. ## Slightly slower compared to N-channel devices. ## Used in complementary circuits with NMOS. ## Combining both classifications gives four main MOSFET types: ## N-Channel Enhancement MOSFET ## P-Channel Enhancement MOSFET ## N-Channel Depletion MOSFET ## P-Channel Depletion MOSFET ## 2. Applications of MOSFET ## MOSFETs are widely used in electronic circuits because of their high input impedance, fast switching speed, and low power consumption. ## Some common applications include: ## Digital Circuits ## Used in CMOS logic gates, microprocessors, and memory devices. ## Power Electronics ## Used as switches in power supplies, inverters, and motor drivers. ## Amplifiers ## Switching Applications ## Analog Circuits ## Used in operational amplifiers and signal processing circuits.

## Construct complete structure of Enhancement and Depletion type MOSFETs