## Pre-Lab Task

## Task 1:

1. What is the significance of small signal analysis in transistor amplifiers?

Small signal analysis is important because it lets us study how a transistor amplifier behaves when a tiny AC signal is applied around its operating point (Q-point). In real circuits, the input signal is usually very small compared to the DC bias, so instead of analyzing the full nonlinear transistor equations (which are complicated), we linearize the circuit around the bias point.

This helps us:

- Find the voltage gain, current gain, and input/output impedance

- Understand how the amplifier will respond to actual signals (like audio)

- Design the amplifier for maximum amplification without distortion

- Predict performance using simple equivalent models (like the hybrid-π model)

2. Why are Coupling and Bypass Capacitors used? Discuss their behavior during AC and DC input signals.

Coupling capacitors and bypass capacitors are used to control how AC and DC components behave in an amplifier.

Coupling Capacitors

Purpose:
They are used to pass the AC signal from one stage to another while blocking DC.

Behavior:

- For DC: They act like an open circuit, so DC biasing of one stage does not disturb the next stage.

- For AC: They act like a short circuit (at mid/high frequencies), allowing the signal to pass through.

This keeps the biasing stable while still transferring the amplified signal.

Bypass Capacitors

Purpose:
They are used to increase the AC gain of the amplifier.

Behavior:

- For DC: Open circuit → does not affect the DC emitter resistance, so biasing remains stable.

- For AC: Provides a low-reactance path to ground, effectively removing the emitter resistor from the AC circuit.

3. List down the advantages and applications of CE Amplifiers.

Advantages of Common Emitter (CE) Amplifier:

- High voltage gain

- High current gain

- High power gain

- Moderate input and output impedance (good for general use)

- Good frequency response for audio range

- Provides 180° phase shift between input and output

Applications:

- Audio amplifiers (microphone and speaker circuits)

- Radio and TV signal amplification

- Voltage amplification stages in multistage amplifiers

- Sensor signal amplification

- General purpose analog amplification