# SYED SUALEH ABBAS 10303

# SYED MOHSIN NAQVI

# SYED ZAID NAQVI

# Lab 6: Design and FPGA Implementation of the ALU for a Single-Cycle RISC-V Processor

# Task 1: ALU Block Diagram Design

# Block diagram of the ALU.

# Task 2: Verilog Description and Testbench Development

## Verilog source file for ALU

## `timescale 1ns / 1ps

## //////////////////////////////////////////////////////////////////////////////////

## // Company:

## // Engineer:

## //

## // Create Date: 02/17/2026

## // Design Name: 32-bit ALU

## // Module Name: ALU32bit

## // Description: Self-contained 32-bit ALU with multiple operations

## //////////////////////////////////////////////////////////////////////////////////

## module ALU32bit(

## input  [31:0] A,          // First operand

## input  [31:0] B,          // Second operand

## input  [3:0]  ALUControl, // Control signals to select operation

## output reg [31:0] ALUResult,  // Result of ALU

## output Zero                // Zero flag

## );

## // Internal wires

## wire [31:0] A_in, B_in;

## wire [31:0] and_out, or_out, xor_out, nor_out, sll_out, srl_out, sum, slt_out;

## wire [31:0] carry;

## // Invert inputs if needed (for SUB or NOR)

## assign A_in = (ALUControl[3]) ? ~A : A;

## assign B_in = (ALUControl[2]) ? ~B : B;

## // Logic operations

## assign and_out = A_in & B_in;

## assign or_out  = A_in | B_in;

## assign xor_out = A_in ^ B_in;

## assign nor_out = ~(A_in | B_in);

## // Shift operations

## assign sll_out = A << B[4:0]; // Logical left shift

## assign srl_out = A >> B[4:0]; // Logical right shift

## // Ripple carry adder for ADD/SUB

## assign sum[0] = A_in[0] ^ B_in[0] ^ ALUControl[2]; // cin = 1 for subtraction

## assign carry[0] = (A_in[0] & B_in[0]) | (B_in[0] & ALUControl[2]) | (A_in[0] & ALUControl[2]);

## genvar i;

## generate

## for (i = 1; i < 32; i = i + 1) begin : adder

## assign sum[i] = A_in[i] ^ B_in[i] ^ carry[i-1];

## assign carry[i] = (A_in[i] & B_in[i]) | (B_in[i] & carry[i-1]) | (A_in[i] & carry[i-1]);

## end

## endgenerate

## // SLT (Set on Less) - only LSB = 1 if A < B

## assign slt_out = 32'b0 | (sum[31] ? 32'b1 : 32'b0);

## // Output multiplexer based on ALUControl

## always @(*) begin

## case(ALUControl)

## 4'b0000: ALUResult = and_out;   // AND

## 4'b0001: ALUResult = or_out;    // OR

## 4'b0010: ALUResult = sum;       // ADD

## 4'b0110: ALUResult = sum;       // SUB

## 4'b0011: ALUResult = xor_out;   // XOR

## 4'b1100: ALUResult = nor_out;   // NOR

## 4'b0111: ALUResult = slt_out;   // SLT

## 4'b0100: ALUResult = sll_out;   // SLL

## 4'b0101: ALUResult = srl_out;   // SRL

## default: ALUResult = 32'b0;

## endcase

## end

## // Zero flag

## assign Zero = (ALUResult == 32'b0);

## endmodule

## Testbench file

## `timescale 1ns / 1ps

## //////////////////////////////////////////////////////////////////////////////////

## // Testbench for ALU32bit

## //////////////////////////////////////////////////////////////////////////////////

## module ALU32bit_tb;

## // Inputs

## reg [31:0] A;

## reg [31:0] B;

## reg [3:0] ALUControl;

## // Outputs

## wire [31:0] ALUResult;

## wire Zero;

## // Instantiate the ALU

## ALU32bit uut (

## .A(A),

## .B(B),

## .ALUControl(ALUControl),

## .ALUResult(ALUResult),

## .Zero(Zero)

## );

## initial begin

## // Initialize inputs

## A = 32'd15;

## B = 32'd10;

## // Test AND

## ALUControl = 4'b0000; // AND

## #10;

## // Test OR

## ALUControl = 4'b0001; // OR

## #10;

## // Test ADD

## ALUControl = 4'b0010; // ADD

## #10;

## // Test SUB

## ALUControl = 4'b0110; // SUB

## #10;

## // Test XOR

## ALUControl = 4'b0011; // XOR

## #10;

## // Test NOR

## ALUControl = 4'b1100; // NOR

## #10;

## // Test SLT (A < B)

## A = 32'd5;

## B = 32'd10;

## ALUControl = 4'b0111; // SLT

## #10;

## // Test SLT (A > B)

## A = 32'd20;

## B = 32'd10;

## ALUControl = 4'b0111; // SLT

## #10;

## // Test SLL (shift left)

## A = 32'h0000_000F; // 15

## B = 32'd2;         // shift by 2

## ALUControl = 4'b0100; // SLL

## #10;

## // Test SRL (shift right)

## A = 32'h0000_00F0; // 240

## B = 32'd4;         // shift by 4

## ALUControl = 4'b0101; // SRL

## #10;

## // Test Zero flag

## A = 32'd5;

## B = 32'd5;

## ALUControl = 4'b0110; // SUB → 0

## #10;

## $finish;

## end

## endmodule

## Simulation waveforms showing correct operation

and

OR

ADD

SUB

XOR

NOR

SLT

SLL

SRL

Truth table of test bench:

# Task 3: FPGA Synthesis and RTL Analysis

## RTL schematic screenshots

## Demonstration of FPGA operation

FPGA  PICTURES:

AND:

ADD:

OR:

XOR:

SUB:

# Assessment Rubric

Lab 6: Design and FPGA Implementation of the ALU for a Single-Cycle RISC-V Processor

| Name: | Student ID: | Section: |
| --- | --- | --- |

Points Distribution

|  | Task No. | LR 2 (Code) | LR 5 (Results) | LR 11 (Design) |
| --- | --- | --- | --- | --- |
| In-Lab | Task 1 | - | - | /20 |
|  | Task 2 | /20 | /15 | - |
|  | Task 3 | /10 | /15 | - |
| Total Points |  | /30 | /30 | /20 |
| CLO Mapped |  | CLO 1 |  |  |

| Affective Domain Rubric |  | Points | CLO Mapped |
| --- | --- | --- | --- |
| AR7 | Report Submission & Git Upload | /10 & /10 | CLO 1 |

| CLO | Total Points | Points Obtained |
| --- | --- | --- |
| 1 | 100 |  |
| Total | 100 |  |

For description of different levels of the mapped rubrics, please refer to the Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics provided here.