# Syed Sualeh abbas 10303

# Syed Mohsin Mujtaba Rizvi mm10016

# Syed zain naqvi

# Lab 7: Design and FPGA Implementation of the Register File for a Single-Cycle RISC-V Processor

Objective

# Task 1: Register File Block Diagram Design

# Block diagram of Register File

# Task 2: Verilog Description and Testbench Development

## Verilog source file for Register File

TEST BENCH CODE:

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Design Name: RF + ALU + FSM Testbench

// Module Name: RF_ALU_FSM_tb

// Description: Thorough testbench that writes multiple registers,

//              performs multiple ALU operations, prints all signals

//////////////////////////////////////////////////////////////////////////////////

module RF_ALU_FSM_tb;

// Clock and reset

reg clk;

reg rst;

// Register File signals

reg writeEnable;

reg [4:0] rs1, rs2, rd;   // read/write addresses

reg [31:0] writeData;

wire [31:0] readData1, readData2;

// ALU signals

reg [3:0] ALUControl;

wire [31:0] ALUResult;

wire Zero;

reg [31:0] A, B;  // ALU inputs

// FSM state definition

reg [3:0] state;

localparam IDLE           = 0,

WRITE_REGS     = 1,

ALU_OP         = 2,

WRITE_ALU_RES  = 3,

NEXT_ALU_OP    = 4,

DONE           = 5;

integer i;

// Instantiate Register File

RegisterFile RF(

.clk(clk),

.rst(rst),

.writeEnable(writeEnable),

.readAddr1(rs1),

.readAddr2(rs2),

.writeAddr(rd),

.writeData(writeData),

.readData1(readData1),

.readData2(readData2)

);

// Instantiate ALU

ALU32bit ALU(

.A(A),

.B(B),

.ALUControl(ALUControl),

.ALUResult(ALUResult),

.Zero(Zero)

);

// Clock generation: 10ns period

initial clk = 0;

always #5 clk = ~clk;

// Arrays of test data

reg [31:0] regValues [1:10];   // Values to write

reg [4:0] regDest [1:10];      // Destination registers

reg [3:0] aluOps [0:7];        // ALU operations

initial begin

// Example register values

regValues[1] = 32'h10101010;

regValues[2] = 32'h01010101;

regValues[3] = 32'h00000005;

regValues[4] = 32'hFFFFFFFF;

regValues[5] = 32'h0000FFFF;

regValues[6] = 32'hAAAA5555;

regValues[7] = 32'h12345678;

regValues[8] = 32'h87654321;

regValues[9] = 32'h0F0F0F0F;

regValues[10] = 32'hF0F0F0F0;

// Destination registers to write ALU results

regDest[1] = 4;

regDest[2] = 5;

regDest[3] = 6;

regDest[4] = 7;

regDest[5] = 8;

regDest[6] = 9;

regDest[7] = 10;

regDest[8] = 11;

regDest[9] = 12;

regDest[10] = 13;

// ALU operations

aluOps[0] = 4'b0010; // ADD

aluOps[1] = 4'b0110; // SUB

aluOps[2] = 4'b0000; // AND

aluOps[3] = 4'b0001; // OR

aluOps[4] = 4'b0011; // XOR

aluOps[5] = 4'b0100; // SLL

aluOps[6] = 4'b0101; // SRL

aluOps[7] = 4'b0111; // SLT

end

// FSM sequence

reg [3:0] aluIndex;

reg [3:0] writeIndex;

always @(posedge clk or posedge rst) begin

if (rst) begin

state <= IDLE;

writeEnable <= 0;

rs1 <= 0; rs2 <= 0; rd <= 0;

writeData <= 0;

ALUControl <= 0;

writeIndex <= 1;

aluIndex <= 0;

end else begin

case(state)

IDLE: state <= WRITE_REGS;

// Write multiple registers

WRITE_REGS: begin

if (writeIndex <= 10) begin

writeEnable <= 1;

rd <= writeIndex;

writeData <= regValues[writeIndex];

writeIndex <= writeIndex + 1;

end else begin

writeEnable <= 0;

writeIndex <= 1;

state <= ALU_OP;

end

end

// ALU operations between x1 and x2 repeatedly

ALU_OP: begin

rs1 <= 1; rs2 <= 2;

A <= readData1; B <= readData2;

ALUControl <= aluOps[aluIndex];

state <= WRITE_ALU_RES;

end

WRITE_ALU_RES: begin

writeEnable <= 1;

rd <= regDest[aluIndex+1];

writeData <= ALUResult;

state <= NEXT_ALU_OP;

end

NEXT_ALU_OP: begin

writeEnable <= 0;

if (aluIndex < 7) begin

aluIndex <= aluIndex + 1;

state <= ALU_OP;

end else begin

state <= DONE;

end

end

DONE: begin

$display("Test Completed!");

$finish;

end

endcase

end

end

// Monitor all outputs

initial begin

$monitor("Time=%0t | State=%0d | rs1=%0d rs2=%0d rd=%0d | writeEn=%b | readData1=%h readData2=%h | A=%h B=%h | ALUResult=%h Zero=%b",

$time, state, rs1, rs2, rd, writeEnable, readData1, readData2, A, B, ALUResult, Zero);

end

// Waveform dump for GTKWave or ModelSim

initial begin

$dumpfile("RF_ALU_FSM_tb.vcd");

$dumpvars(0, RF_ALU_FSM_tb);

end

// Initial reset

initial begin

rst = 1;

#10 rst = 0;

end

endmodule

## Verilog source file for Top-level (top rf alu.v).

## `timescale 1ns / 1ps

## module TOPmodule_registers(

## input clk,

## input rst,

## input [15:0] switches,

## output [15:0] leds

## );

## // ===============================

## // WIRES

## // ===============================

## wire [31:0] readData1;

## wire [31:0] readData2;

## wire [31:0] ALUResult;

## wire Zero;

## reg writeEnable;

## reg [4:0] writeAddr;

## wire [31:0] writeData;

## // ===============================

## // CONSTANTS (for quick verification)

## // ===============================

## parameter CONST_A = 32'h10101010;

## parameter CONST_B = 32'h01010101;

## // ===============================

## // WRITE ADDRESS CONTROL

## // ===============================

## always @(*) begin

## case(switches[6:5])

## 2'b01: writeAddr = 5'd1;   // load x1

## 2'b10: writeAddr = 5'd2;   // load x2

## 2'b11: writeAddr = 5'd3;   // store ALU result

## default: writeAddr = 5'd0;

## endcase

## end

## // ===============================

## // WRITE ENABLE

## // ===============================

## always @(*) begin

## writeEnable = switches[0];

## end

## // ===============================

## // WRITE DATA MUX

## // ===============================

## assign writeData =

## (switches[6:5] == 2'b01) ? CONST_A :

## (switches[6:5] == 2'b10) ? CONST_B :

## (switches[6:5] == 2'b11) ? ALUResult :

## 32'b0;

## // ===============================

## // REGISTER FILE

## // ===============================

## RegisterFile RF (

## .clk(clk),

## .rst(rst),

## .writeEnable(writeEnable),

## .readAddr1(5'd1),   // always read x1

## .readAddr2(5'd2),   // always read x2

## .writeAddr(writeAddr),

## .writeData(writeData),

## .readData1(readData1),

## .readData2(readData2)

## );

## // ===============================

## // ALU

## // ===============================

## ALU32bit ALU (

## .A(readData1),

## .B(readData2),

## .ALUControl(switches[4:1]),

## .ALUResult(ALUResult),

## .Zero(Zero)

## );

## // ===============================

## // LED OUTPUT

## // ===============================

## assign leds[15] = Zero;                 // show Zero flag

## assign leds[14:0] = ALUResult[14:0];    // show result LSBs

## endmodule

## Testbench file for Top-level

## `timescale 1ns / 1ps

## module TOPmodule_registers_tb;

## reg clk;

## reg rst;

## reg [15:0] switches;

## wire [15:0] leds;

## // Instantiate DUT

## TOPmodule_registers DUT (

## .clk(clk),

## .rst(rst),

## .switches(switches),

## .leds(leds)

## );

## // Clock generation (10ns period)

## always #5 clk = ~clk;

## // =============================

## // TASK: Perform a write

## // =============================

## task write_operation;

## input [1:0] addrSel;

## begin

## switches[6:5] = addrSel; // select x1, x2, or x3

## switches[0]   = 1;       // write enable

## @(posedge clk);

## #2 switches[0] = 0;      // disable write

## end

## endtask

## // =============================

## // TASK: Perform ALU operation

## // =============================

## task alu_operation;

## input [3:0] aluCtrl;

## begin

## switches[4:1] = aluCtrl;

## @(posedge clk);

## #2;

## $display("TIME=%0t | ALUControl=%b | LEDS=%h | ZERO=%b",

## $time, aluCtrl, leds, leds[15]);

## end

## endtask

## // =============================

## // TEST SEQUENCE

## // =============================

## initial begin

## $display("===== STARTING SIMULATION =====");

## clk = 0;

## rst = 1;

## switches = 0;

## #20;

## rst = 0;

## // -----------------------------

## // Load CONST_A into x1

## // -----------------------------

## $display("Loading CONST_A into x1");

## write_operation(2'b01);

## // -----------------------------

## // Load CONST_B into x2

## // -----------------------------

## $display("Loading CONST_B into x2");

## write_operation(2'b10);

## // -----------------------------

## // ALU OPERATIONS

## // -----------------------------

## alu_operation(4'b0010); // ADD

## alu_operation(4'b0110); // SUB

## alu_operation(4'b0000); // AND

## alu_operation(4'b0001); // OR

## alu_operation(4'b0011); // XOR

## alu_operation(4'b0111); // SLT

## // -----------------------------

## // Store ALU result into x3

## // -----------------------------

## $display("Storing ALU result into x3");

## write_operation(2'b11);

## #50;

## $display("===== SIMULATION FINISHED =====");

## $stop;

## end

## endmodule

## Simulation waveforms showing correct operation

ALU OPERATION BITS:

Expected Results:

WAVEFORMS:

When NO VALUES ARE ADDED TO REGISTERS FOR ALU INPUTS: (A, B; Both has value of zero)

AFTER WRITE DATA IS TURNED ON THE HARD CODED VALUES GET SAVED IN READ DATA1, READ DATA2

WHEN ALU CONTROL=0, THAT MEANS IT SHOULD PERFORM AND:

When ALU CONTROL=1, THAT MEANS IT SHOULD PERFORM OR:

WHEN ALU CONTROL=5, THAT MEANS IT SHOLD PERFORM SRL:

WHEN ALU CONTROL=7, THAT MEASN IT SHOULD PERFORM SLT:

REST CASES ARE ALSO DONE SAME WAY…..

# Task 3: FPGA Synthesis and RTL Analysis

Note: The switch interface, button debouncing module, and LED interface must be re used from Lab 5.

## RTL schematic screenshots (register file array, decoder, FSM, ALU connections)

## Demonstration of FPGA operation

## VALUE STORD IN A= 0X10101010

## B=0X01010101

NOTE: THE 2 LED’S WHCH ARE NOT WORKING LED: 1, 2; WERE LIGHLTLY LIT BT WERE NO CAPTURED IN THE PICTURE.

# Assessment Rubric

Lab 7: Design and FPGA Implementation of the Register File for a Single-Cycle RISC-V Processor

| Name: | Student ID: | Section: |
| --- | --- | --- |

Points Distribution

|  | Task No. | LR 2 (Code) | LR 5 (Results) | LR 11 (Design) |
| --- | --- | --- | --- | --- |
| In-Lab | Task 1 | - | - | /10 |
|  | Task 2 | /20 | /10 | - |
|  | Task 3 | /20 | /20 | - |
| Total Points |  | /40 | /30 | /10 |
| CLO Mapped |  | CLO 1 |  |  |

| Affective Domain Rubric |  | Points | CLO Mapped |
| --- | --- | --- | --- |
| AR7 | Report Submission & Git Upload | /10 & /10 | CLO 1 |

| CLO | Total Points | Points Obtained |
| --- | --- | --- |
| 1 | 100 |  |
| Total | 100 |  |

For description of different levels of the mapped rubrics, please refer to the Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics provided here.