# Syed Sualeh Abbas

# Syed Zaid Naqvi

# Syed Mohsin

# Lab 5: Designing FSM Using FPGA Switches and LEDs

# Task 1:

## FSM Design and State Diagram

# Task 2: Verilog FSM and Counter Implementation

## Verilog source file for FSM and counter

## `timescale 1ns / 1ps

## module debouncher(

## input  wire clk,

## input  wire pbin,

## output reg  pbout

## );

## // Debounce using a counter: output changes only when input stays stable long enough

## // With 100 MHz, COUNT_MAX=2_000_000 ~ 20ms

## parameter integer COUNT_MAX = 2_000_000;

## reg sync1 = 0, sync2 = 0;

## reg [31:0] cnt = 0;

## reg stable = 0;

## always @(posedge clk) begin

## // 2-flop synchronizer

## sync1 <= pbin;

## sync2 <= sync1;

## if (sync2 == stable) begin

## cnt <= 0; // already stable

## end else begin

## if (cnt == COUNT_MAX-1) begin

## stable <= sync2;

## cnt <= 0;

## end else begin

## cnt <= cnt + 1;

## end

## end

## pbout <= stable;

## end

## endmodule

## `timescale 1ns / 1ps

## module switches(

## input  wire        clk, rst,

## input  wire [15:0] btns,        // not used here

## input  wire [31:0] writeData,   // not used

## input  wire        writeEnable, // not used

## input  wire        readEnable,

## input  wire [29:0] memAddress,

## input  wire [15:0] switches,

## output reg  [31:0] readData

## );

## reg [7:0] switchData [3:0];

## // Address for switches peripheral

## localparam [29:0] SW_ADDR = 30'h00000001;

## always @(posedge clk) begin

## if (rst) begin

## readData <= 32'd0;

## switchData[0] <= 8'd0;

## switchData[1] <= 8'd0;

## switchData[2] <= 8'd0;

## switchData[3] <= 8'd0;

## end else begin

## // store switch bytes (optional, matches your skeleton)

## switchData[0] <= switches[7:0];

## switchData[1] <= switches[15:8];

## switchData[2] <= 8'd0;

## switchData[3] <= 8'd0;

## // read transaction

## if (readEnable && memAddress == SW_ADDR)

## readData <= {16'd0, switches};

## else

## readData <= 32'd0;

## end

## end

## endmodule

## `timescale 1ns / 1ps

## module leds(

## input  wire        clk,

## input  wire        rst,

## input  wire [31:0] writeData,

## input  wire        writeEnable,

## input  wire        readEnable,   // not used

## input  wire [29:0] memAddress,

## output reg  [31:0] readData = 0, // not to be read

## output reg  [15:0] leds

## );

## reg [7:0] ledData [3:0];

## // Address for LED peripheral

## localparam [29:0] LED_ADDR = 30'h00000002;

## integer i;

## always @(posedge clk) begin

## if (rst) begin

## leds <= 16'd0;

## for (i = 0; i < 4; i = i + 1)

## ledData[i] <= 8'd0;

## end else begin

## if (writeEnable && memAddress == LED_ADDR) begin

## ledData[0] <= writeData[7:0];

## ledData[1] <= writeData[15:8];

## ledData[2] <= writeData[23:16];

## ledData[3] <= writeData[31:24];

## leds <= writeData[15:0];

## end

## end

## end

## endmodule

## `timescale 1ns / 1ps

## module Task01_using_peripherals #(

## parameter integer DIV_MAX = 100_000_000

## )(

## input  wire        clk,

## input  wire        btnC,       // RESET raw

## input  wire [15:0] sw,         // physical switches

## output wire [6:0]  seg,

## output wire [3:0]  an,

## output wire [15:0] led         // driven by leds module

## );

## // -----------------------------

## // Debounced reset

## // -----------------------------

## wire rst;

## debouncher u_db (

## .clk(clk),

## .pbin(btnC),

## .pbout(rst)

## );

## // -----------------------------

## // MMIO addressing

## // -----------------------------

## localparam [29:0] SW_ADDR  = 30'h00000001;

## localparam [29:0] LED_ADDR = 30'h00000002;

## // Switch peripheral signals

## reg         sw_readEnable;

## wire [31:0] sw_readData;

## // LED peripheral signals

## reg  [31:0] led_writeData;

## reg         led_writeEnable;

## // Instantiate peripherals

## switches u_switches (

## .clk(clk),

## .rst(rst),

## .btns(16'd0),

## .writeData(32'd0),

## .writeEnable(1'b0),

## .readEnable(sw_readEnable),

## .memAddress(SW_ADDR),

## .switches(sw),

## .readData(sw_readData)

## );

## leds u_leds (

## .clk(clk),

## .rst(rst),

## .writeData(led_writeData),

## .writeEnable(led_writeEnable),

## .readEnable(1'b0),

## .memAddress(LED_ADDR),

## .readData(),

## .leds(led)

## );

## // -----------------------------

## // 1 Hz tick generator

## // -----------------------------

## reg [31:0] div;

## wire tick_1hz = (div == (DIV_MAX-1));

## always @(posedge clk) begin

## if (rst)

## div <= 0;

## else if (tick_1hz)

## div <= 0;

## else

## div <= div + 1;

## end

## // -----------------------------

## // Priority encoder

## // -----------------------------

## function [3:0] encode;

## input [15:0] s;

## begin

## casex (s)

## 16'b1xxx_xxxx_xxxx_xxxx: encode = 4'd15;

## 16'b01xx_xxxx_xxxx_xxxx: encode = 4'd14;

## 16'b001x_xxxx_xxxx_xxxx: encode = 4'd13;

## 16'b0001_xxxx_xxxx_xxxx: encode = 4'd12;

## 16'b0000_1xxx_xxxx_xxxx: encode = 4'd11;

## 16'b0000_01xx_xxxx_xxxx: encode = 4'd10;

## 16'b0000_001x_xxxx_xxxx: encode = 4'd9;

## 16'b0000_0001_xxxx_xxxx: encode = 4'd8;

## 16'b0000_0000_1xxx_xxxx: encode = 4'd7;

## 16'b0000_0000_01xx_xxxx: encode = 4'd6;

## 16'b0000_0000_001x_xxxx: encode = 4'd5;

## 16'b0000_0000_0001_xxxx: encode = 4'd4;

## 16'b0000_0000_0000_1xxx: encode = 4'd3;

## 16'b0000_0000_0000_01xx: encode = 4'd2;

## 16'b0000_0000_0000_001x: encode = 4'd1;

## default:                 encode = 4'd0;

## endcase

## end

## endfunction

## // Read switches continuously (MMIO read)

## reg [15:0] sw_sample;

## reg [15:0] sw_prev;

## wire [3:0] preview_value = encode(sw_sample);

## wire any_sw_rise = |(sw_sample & ~sw_prev);

## always @(posedge clk) begin

## if (rst) begin

## sw_readEnable <= 1'b0;

## sw_sample     <= 16'd0;

## sw_prev       <= 16'd0;

## end else begin

## sw_readEnable <= 1'b1;

## sw_prev       <= sw_sample;

## sw_sample     <= sw_readData[15:0];

## end

## end

## // -----------------------------

## // FSM (same behavior as your Task01)

## // -----------------------------

## localparam IDLE  = 2'b00;

## localparam COUNT = 2'b01;

## localparam DONE  = 2'b10;

## reg [1:0] state;

## reg [3:0] counter;

## always @(posedge clk) begin

## if (rst) begin

## state   <= IDLE;

## counter <= 4'd0;

## end else begin

## case (state)

## IDLE: begin

## counter <= preview_value; // preview

## if (any_sw_rise && (preview_value != 4'd0)) begin

## counter <= preview_value; // latch once

## state   <= COUNT;

## end

## end

## COUNT: begin

## if (tick_1hz) begin

## if (counter > 0)

## counter <= counter - 1;

## else

## state <= DONE;

## end

## end

## DONE: begin

## counter <= 4'd0;

## if (any_sw_rise && (preview_value != 4'd0)) begin

## counter <= preview_value;

## state   <= COUNT;

## end

## end

## endcase

## end

## end

## // -----------------------------

## // Drive LED peripheral (MMIO write)

## // same mapping as before:

## // led[3:0]=counter, led[5:4]=state, led[9:6]=preview

## // -----------------------------

## always @(posedge clk) begin

## if (rst) begin

## led_writeEnable <= 1'b0;

## led_writeData   <= 32'd0;

## end else begin

## led_writeEnable <= 1'b1;

## led_writeData   <= {22'd0, preview_value, state, counter};

## end

## end

## // -----------------------------

## // 7-seg HEX (0..F) rightmost digit

## // -----------------------------

## assign an = 4'b1110;

## function [6:0] hex7seg;

## input [3:0] x;

## begin

## case (x)

## 4'h0: hex7seg = 7'b1000000;

## 4'h1: hex7seg = 7'b1111001;

## 4'h2: hex7seg = 7'b0100100;

## 4'h3: hex7seg = 7'b0110000;

## 4'h4: hex7seg = 7'b0011001;

## 4'h5: hex7seg = 7'b0010010;

## 4'h6: hex7seg = 7'b0000010;

## 4'h7: hex7seg = 7'b1111000;

## 4'h8: hex7seg = 7'b0000000;

## 4'h9: hex7seg = 7'b0010000;

## 4'hA: hex7seg = 7'b0001000;

## 4'hB: hex7seg = 7'b0000011;

## 4'hC: hex7seg = 7'b1000110;

## 4'hD: hex7seg = 7'b0100001;

## 4'hE: hex7seg = 7'b0000110;

## 4'hF: hex7seg = 7'b0001110;

## endcase

## end

## endfunction

## assign seg = hex7seg(counter);

## endmodule

## Testbench file for FSM and counter

## `timescale 1ns / 1ps

## module Task01_using_peripherals_tb;

## // -------------------------

## // Parameters - match the REAL hardware values exactly

## // No parameter overrides needed; design runs as on board

## // -------------------------

## localparam integer CLK_PERIOD  = 10;          // 10ns = 100 MHz

## localparam integer DB_CYCLES   = 2_000_000;   // debouncer COUNT_MAX

## localparam integer TICK_CYCLES = 100_000_000; // 1Hz tick DIV_MAX

## // -------------------------

## // DUT signals

## // -------------------------

## reg         clk;

## reg         btnC;

## reg  [15:0] sw;

## wire [6:0]  seg;

## wire [3:0]  an;

## wire [15:0] led;

## // -------------------------

## // Instantiate DUT - no overrides, pure hardware config

## // -------------------------

## Task01_using_peripherals dut (

## .clk (clk),

## .btnC(btnC),

## .sw  (sw),

## .seg (seg),

## .an  (an),

## .led (led)

## );

## // -------------------------

## // Clock generation

## // -------------------------

## initial clk = 0;

## always #(CLK_PERIOD/2) clk = ~clk;

## // -------------------------

## // Helper task: apply reset

## // Holds btnC HIGH for DB_CYCLES+100 clocks so debouncer completes

## // -------------------------

## task do_reset;

## begin

## $display("[RESET] Asserting btnC HIGH for %0d cycles...", DB_CYCLES+100);

## btnC = 1;

## #(CLK_PERIOD * (DB_CYCLES + 100)); // outlast the debouncer

## btnC = 0;

## #(CLK_PERIOD * 100);               // settle

## $display("[RESET] Done. rst should have fired.");

## end

## endtask

## // -------------------------

## // Helper task: press a switch value, hold briefly, then release

## // -------------------------

## task press_sw;

## input [15:0] sw_val;

## input integer hold_cycles;

## begin

## sw = sw_val;

## #(CLK_PERIOD * hold_cycles);

## sw = 16'd0;

## end

## endtask

## // -------------------------

## // Helper: wait for N complete 1Hz ticks

## // One tick = TICK_CYCLES clocks = 1 real second

## // -------------------------

## task wait_ticks;

## input integer n;

## begin

## #(CLK_PERIOD * TICK_CYCLES * n);

## end

## endtask

## // -------------------------

## // Test sequence

## // -------------------------

## initial begin

## // Initialise

## btnC = 0;

## sw   = 16'd0;

## #(CLK_PERIOD * 10);

## // ======================================================

## // RESET - hold btnC long enough for the real debouncer

## // ======================================================

## $display("\n========================================");

## $display("[RESET] Starting reset sequence");

## $display("========================================");

## do_reset;

## $display("[RESET] Expecting: state=IDLE, counter=0, seg=1000000 (digit 0)");

## #(CLK_PERIOD * 50);

## // ======================================================

## // TEST 1: sw[2] only -> priority encode = 2

## // FSM: IDLE -> COUNT(2) -> COUNT(1) -> COUNT(0) -> DONE

## // ======================================================

## $display("\n========================================");

## $display("[Test 1] sw[2] HIGH => priority = 2");

## $display("         Expect: latch 2, count 2->1->0, then DONE");

## $display("========================================");

## press_sw(16'b0000_0000_0000_0100, 10);

## wait_ticks(4);   // 2 ticks to count down + 2 margin

## $display("[Test 1] Done. Expecting state=DONE, counter=0");

## #(CLK_PERIOD * 50);

## // ======================================================

## // TEST 2: sw[15] only -> priority encode = 15

## // ======================================================

## $display("\n========================================");

## $display("[Test 2] sw[15] HIGH => priority = 15");

## $display("         Expect: latch 15, count 15->0, then DONE");

## $display("========================================");

## press_sw(16'b1000_0000_0000_0000, 10);

## wait_ticks(17);  // 15 ticks + 2 margin

## $display("[Test 2] Done. Expecting state=DONE, counter=0");

## #(CLK_PERIOD * 50);

## // ======================================================

## // TEST 3: sw[14] and sw[2] both HIGH -> priority = 14

## // ======================================================

## $display("\n========================================");

## $display("[Test 3] sw[14]+sw[2] => priority = 14 (highest wins)");

## $display("         Expect: latch 14, count 14->0, then DONE");

## $display("========================================");

## press_sw(16'b0100_0000_0000_0100, 10);

## wait_ticks(16);  // 14 ticks + 2 margin

## $display("[Test 3] Done. Expecting state=DONE, counter=0");

## #(CLK_PERIOD * 50);

## // ======================================================

## // TEST 4: Re-trigger from DONE

## // ======================================================

## $display("\n========================================");

## $display("[Test 4] Re-trigger from DONE state");

## $display("         Step A: sw[1] => priority 1, wait for DONE");

## $display("========================================");

## press_sw(16'b0000_0000_0000_0010, 10); // sw[1] = priority 1

## wait_ticks(3);

## $display("[Test 4] In DONE. Now pressing sw[3] to re-trigger...");

## press_sw(16'b0000_0000_0000_1000, 10); // sw[3] = priority 3

## wait_ticks(5);

## $display("[Test 4] Done. Expecting state=DONE, counter=0");

## #(CLK_PERIOD * 50);

## // ======================================================

## // TEST 5: No switch -> stays IDLE

## // ======================================================

## $display("\n========================================");

## $display("[Test 5] No switch pressed -> stays IDLE");

## $display("========================================");

## wait_ticks(3);

## $display("[Test 5] Done. Expecting state=IDLE, counter=0");

## // ======================================================

## // TEST 6: Reset mid-count

## // ======================================================

## $display("\n========================================");

## $display("[Test 6] Reset during COUNT");

## $display("         sw[8] => priority 8, reset after 3 ticks");

## $display("========================================");

## press_sw(16'b0000_0001_0000_0000, 10); // sw[8] = priority 8

## wait_ticks(3);

## $display("[Test 6] Applying reset now (counter should be ~5)...");

## do_reset;

## $display("[Test 6] Done. Expecting state=IDLE, counter=0");

## #(CLK_PERIOD * 50);

## $display("\n========================================");

## $display("===== All Tests Complete =====");

## $display("========================================");

## $finish;

## end

## // -------------------------

## // Decode LED fields

## // led[3:0]  = counter

## // led[5:4]  = state (00=IDLE, 01=COUNT, 10=DONE)

## // led[9:6]  = preview_value

## // -------------------------

## reg [1:0]     state_bits;

## reg [3:0]     counter_bits;

## reg [3:0]     preview_bits;

## reg [8*5-1:0] state_name;

## always @(*) begin

## counter_bits = led[3:0];

## state_bits   = led[5:4];

## preview_bits = led[9:6];

## case (state_bits)

## 2'b00:   state_name = "IDLE ";

## 2'b01:   state_name = "COUNT";

## 2'b10:   state_name = "DONE ";

## default: state_name = "?????";

## endcase

## end

## // -------------------------

## // Print only on changes to avoid billions of identical lines

## // -------------------------

## reg [1:0]  last_state;

## reg [3:0]  last_counter;

## reg [3:0]  last_preview;

## reg        last_btnC;

## reg [15:0] last_sw;

## initial begin

## last_state   = 2'bxx;

## last_counter = 4'bxxxx;

## last_preview = 4'bxxxx;

## last_btnC    = 1'bx;

## last_sw      = 16'bx;

## $display("\n%-14s | %-5s | %-5s | %-16s | %-5s | ctr | prev | seg",

## "Time(ns)", "btnC", "rst", "sw", "state");

## end

## always @(posedge clk) begin

## #1;

## if (state_bits   !== last_state   ||

## counter_bits !== last_counter ||

## preview_bits !== last_preview ||

## btnC         !== last_btnC    ||

## sw           !== last_sw)

## begin

## $display("%-14t | %-5b | %-5b | %b | %s  | %2d  |  %2d  | %7b",

## $time,

## btnC,

## dut.rst,

## sw,

## state_name,

## counter_bits,

## preview_bits,

## seg);

## last_state   <= state_bits;

## last_counter <= counter_bits;

## last_preview <= preview_bits;

## last_btnC    <= btnC;

## last_sw      <= sw;

## end

## end

## endmodule

## Simulation waveforms showing correct operation

# Task 3: FPGA Constraints, XDC File and Timing Analysis

## XDC file

## set_property PACKAGE_PIN E3 [get_ports clk]

## set_property IOSTANDARD LVCMOS33 [get_ports clk]

## create_clock -period 10.000 -name sys_clk [get_ports clk]

## ## Reset (btnC center button)

## set_property PACKAGE_PIN N17 [get_ports btnC]

## set_property IOSTANDARD LVCMOS33 [get_ports btnC]

## ## Switches

## set_property PACKAGE_PIN J15 [get_ports {sw[0]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[0]}]

## set_property PACKAGE_PIN L16 [get_ports {sw[1]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[1]}]

## set_property PACKAGE_PIN M13 [get_ports {sw[2]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[2]}]

## set_property PACKAGE_PIN R15 [get_ports {sw[3]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[3]}]

## set_property PACKAGE_PIN R17 [get_ports {sw[4]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[4]}]

## set_property PACKAGE_PIN T18 [get_ports {sw[5]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[5]}]

## set_property PACKAGE_PIN U18 [get_ports {sw[6]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[6]}]

## set_property PACKAGE_PIN R13 [get_ports {sw[7]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[7]}]

## set_property PACKAGE_PIN T8  [get_ports {sw[8]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[8]}]

## set_property PACKAGE_PIN U8  [get_ports {sw[9]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[9]}]

## set_property PACKAGE_PIN R16 [get_ports {sw[10]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[10]}]

## set_property PACKAGE_PIN T13 [get_ports {sw[11]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[11]}]

## set_property PACKAGE_PIN H6  [get_ports {sw[12]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[12]}]

## set_property PACKAGE_PIN U12 [get_ports {sw[13]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[13]}]

## set_property PACKAGE_PIN U11 [get_ports {sw[14]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[14]}]

## set_property PACKAGE_PIN V10 [get_ports {sw[15]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {sw[15]}]

## ## LEDs

## set_property PACKAGE_PIN H17 [get_ports {led[0]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[0]}]

## set_property PACKAGE_PIN K15 [get_ports {led[1]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[1]}]

## set_property PACKAGE_PIN J13 [get_ports {led[2]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[2]}]

## set_property PACKAGE_PIN N14 [get_ports {led[3]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[3]}]

## set_property PACKAGE_PIN R18 [get_ports {led[4]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[4]}]

## set_property PACKAGE_PIN V17 [get_ports {led[5]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[5]}]

## set_property PACKAGE_PIN U17 [get_ports {led[6]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[6]}]

## set_property PACKAGE_PIN U16 [get_ports {led[7]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[7]}]

## set_property PACKAGE_PIN V16 [get_ports {led[8]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[8]}]

## set_property PACKAGE_PIN T15 [get_ports {led[9]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[9]}]

## set_property PACKAGE_PIN U14 [get_ports {led[10]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[10]}]

## set_property PACKAGE_PIN T16 [get_ports {led[11]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[11]}]

## set_property PACKAGE_PIN V15 [get_ports {led[12]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[12]}]

## set_property PACKAGE_PIN V14 [get_ports {led[13]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[13]}]

## set_property PACKAGE_PIN V12 [get_ports {led[14]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[14]}]

## set_property PACKAGE_PIN V11 [get_ports {led[15]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {led[15]}]

## ## 7-segment display

## set_property PACKAGE_PIN T10 [get_ports {seg[0]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {seg[0]}]

## set_property PACKAGE_PIN R10 [get_ports {seg[1]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {seg[1]}]

## set_property PACKAGE_PIN K16 [get_ports {seg[2]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {seg[2]}]

## set_property PACKAGE_PIN K13 [get_ports {seg[3]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {seg[3]}]

## set_property PACKAGE_PIN P15 [get_ports {seg[4]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {seg[4]}]

## set_property PACKAGE_PIN T11 [get_ports {seg[5]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {seg[5]}]

## set_property PACKAGE_PIN L18 [get_ports {seg[6]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {seg[6]}]

## ## 7-segment anodes

## set_property PACKAGE_PIN J17 [get_ports {an[0]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {an[0]}]

## set_property PACKAGE_PIN J18 [get_ports {an[1]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {an[1]}]

## set_property PACKAGE_PIN T9  [get_ports {an[2]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {an[2]}]

## set_property PACKAGE_PIN J14 [get_ports {an[3]}]

## set_property IOSTANDARD LVCMOS33 [get_ports {an[3]}]

## Screenshot of Vivado Timing Summary

## WNS value and interpretation (PASS / MARGINAL / FAIL)

# Task 4: FPGA Synthesis and On-Board Verification

## RTL schematic screenshots

## Successful FPGA demonstration

# COUNTDOWN FROM 3 TO ZERO

# (Because we turned on 3rd Switch)

# Assessment Rubric

Lab 5: Designing FSM Using FPGA Switches and LEDs

| Name: | Student ID: | Section: |
| --- | --- | --- |

Points Distribution

|  | Task No. | LR 2 (Code) | LR 5 (Results) | LR 11 (Design) |
| --- | --- | --- | --- | --- |
| In-Lab | Pre lab | - | /15 | - |
|  | Task 1 | - | - | /10 |
|  | Task 2 | /15 | /15 | - |
|  | Task 3 | - | /10 | - |
|  | Task 4 | - | /15 |  |
| Total Points |  | /15 | /55 | /10 |
| CLO Mapped |  | CLO 1 |  |  |

| Affective Domain Rubric |  | Points | CLO Mapped |
| --- | --- | --- | --- |
| AR7 | Report Submission & Git Upload | /10 & /10 | CLO 1 |

| CLO | Total Points | Points Obtained |
| --- | --- | --- |
| 1 | 100 |  |
| Total | 100 |  |

For description of different levels of the mapped rubrics, please refer to the Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics provided here.