# Syed Sualeh abbas, Syed Zaid Naqvi, Mohsin Mujtaba

# Lab 8: Design and FPGA Implementation of Memory System with Address Decoding for a Single-Cycle RISC-V Processor

# Task 1: Design of Data Memory and Address Decoder

# Block diagram of memory system with address decoding.

# Task 2: Verilog Implementation and Testbench Development

## Verilog source file for Data Memory

## module DataMemory (

## input         clk,

## input         MemWrite,   // Write enable (synchronous)

## input         MemRead,    // Read  enable (asynchronous)

## input  [8:0]  address,    // 9-bit local address → 512 locations

## input  [31:0] write_data,

## output reg [31:0] read_data

## );

## // 512-word × 32-bit memory array

## reg [31:0] memory [0:511];

## // Initialise all locations to 0 (helps simulation)

## integer i;

## initial begin

## for (i = 0; i < 512; i = i + 1)

## memory[i] = 32'b0;

## end

## // Synchronous write

## always @(posedge clk) begin

## if (MemWrite)

## memory[address] <= write_data;

## end

## // Asynchronous (combinational) read

## always @(*) begin

## if (MemRead)

## read_data = memory[address];

## else

## read_data = 32'b0;

## end

## endmodule

## Verilog source file for Address Decoder

## module AddressDecoder (

## input  [31:0] address,

## input         readEnable,

## input         writeEnable,

## output reg    DataMemWrite,

## output reg    DataMemRead,

## output reg    LEDWrite,

## output reg    SwitchReadEnable

## );

## // Only the top 2 bits of the 10-bit local address determine the device

## wire [1:0] deviceSelect;

## assign deviceSelect = address[9:8];

## always @(*) begin

## // Safe defaults - only one device active at a time

## DataMemWrite     = 1'b0;

## DataMemRead      = 1'b0;

## LEDWrite         = 1'b0;

## SwitchReadEnable = 1'b0;

## case (deviceSelect)

## 2'b00: begin  // Data Memory (0-511)

## DataMemWrite = writeEnable;

## DataMemRead  = readEnable;

## end

## 2'b01: begin  // LEDs (512-767) - write only

## LEDWrite = writeEnable;

## end

## 2'b10: begin  // Switches (768-1023) - read only

## SwitchReadEnable = readEnable;

## end

## default: begin  // 2'b11 - no device selected

## DataMemWrite     = 1'b0;

## DataMemRead      = 1'b0;

## LEDWrite         = 1'b0;

## SwitchReadEnable = 1'b0;

## end

## endcase

## end

## endmodule

## Testbench

## module MemorySystem_tb;

## // ----------------------------------------------------------------

## // DUT ports

## // ----------------------------------------------------------------

## reg         clk, rst;

## reg  [31:0] address;

## reg         readEnable, writeEnable;

## reg  [31:0] writeData;

## reg  [15:0] switches;

## wire [31:0] readData;

## wire [15:0] leds;

## // ----------------------------------------------------------------

## // Hierarchical probes - visible in the waveform viewer

## // ----------------------------------------------------------------

## wire DataMemWrite     = DUT.decoder.DataMemWrite;

## wire DataMemRead      = DUT.decoder.DataMemRead;

## wire LEDWrite         = DUT.decoder.LEDWrite;

## wire SwitchReadEnable = DUT.decoder.SwitchReadEnable;

## // ----------------------------------------------------------------

## // DUT instantiation

## // ----------------------------------------------------------------

## addressDecoderTop DUT (

## .clk        (clk),

## .rst        (rst),

## .address    (address),

## .readEnable (readEnable),

## .writeEnable(writeEnable),

## .writeData  (writeData),

## .switches   (switches),

## .readData   (readData),

## .leds       (leds)

## );

## // ----------------------------------------------------------------

## // Clock: 10 ns period (100 MHz)

## // ----------------------------------------------------------------

## initial clk = 0;

## always  #5 clk = ~clk;

## // ----------------------------------------------------------------

## // Pass / Fail counters

## // ----------------------------------------------------------------

## integer pass_cnt = 0;

## integer fail_cnt = 0;

## // ----------------------------------------------------------------

## // TASK: check - compare expected vs actual and print result

## // ----------------------------------------------------------------

## task check;

## input [159:0] label;

## input [31:0]  expected;

## input [31:0]  actual;

## begin

## if (expected === actual) begin

## $display("  PASS  %-22s  exp=%h  got=%h",

## label, expected, actual);

## pass_cnt = pass_cnt + 1;

## end else begin

## $display("  FAIL  %-22s  exp=%h  got=%h  <---",

## label, expected, actual);

## fail_cnt = fail_cnt + 1;

## end

## end

## endtask

## // ----------------------------------------------------------------

## // TASK: do_write - synchronous write (one clock cycle)

## //   Drives on negedge so inputs are stable before posedge capture.

## // ----------------------------------------------------------------

## task do_write;

## input [31:0] addr;

## input [31:0] data;

## begin

## @(negedge clk);

## address     = addr;

## writeData   = data;

## writeEnable = 1'b1;

## readEnable  = 1'b0;

## @(posedge clk); #1;   // data captured on this rising edge

## writeEnable = 1'b0;

## end

## endtask

## // ----------------------------------------------------------------

## // TASK: do_read - combinational read

## //   Asserts readEnable, waits 2 ns for combinational settle,

## //   caller samples signals, then deasserts after next posedge.

## // ----------------------------------------------------------------

## task do_read;

## input [31:0] addr;

## begin

## @(negedge clk);

## address     = addr;

## readEnable  = 1'b1;

## writeEnable = 1'b0;

## #2;   // combinational paths settle (well within half-period)

## // *** caller checks readData and decoder signals HERE ***

## @(posedge clk); #1;

## readEnable  = 1'b0;

## end

## endtask

## // ================================================================

## // STIMULUS

## // ================================================================

## initial begin

## $display("====================================================");

## $display("  MemorySystem Testbench - Lab 8");

## $display("====================================================");

## // ---------- Reset ----------

## rst         = 1'b1;

## address     = 32'd0;

## readEnable  = 1'b0;

## writeEnable = 1'b0;

## writeData   = 32'd0;

## switches    = 16'd0;

## repeat (4) @(posedge clk);

## @(negedge clk);

## rst = 1'b0;

## // ============================================================

## // 1. DATA MEMORY  -  address[9:8] = 2'b00  (addresses 0-255)

## // ============================================================

## $display("\n[1] Data Memory - Writes");

## do_write(32'd0,   32'hAAAA_BBBB);

## do_write(32'd1,   32'h1234_5678);

## do_write(32'd10,  32'hDEAD_BEEF);

## do_write(32'd200, 32'hCAFE_F00D);

## do_write(32'd255, 32'h0000_FFFF);  // last address in DataMem range

## $display("\n[2] Data Memory - Read-Back");

## do_read(32'd0);

## check("DM[0]=AAAABBBB",   32'hAAAA_BBBB, readData);

## check("DataMemRead  hi",  32'd1,          DataMemRead);

## check("DataMemWrite lo",  32'd0,          DataMemWrite);

## check("LEDWrite     lo",  32'd0,          LEDWrite);

## check("SwitchRdEn   lo",  32'd0,          SwitchReadEnable);

## do_read(32'd1);

## check("DM[1]=12345678",   32'h1234_5678,  readData);

## do_read(32'd10);

## check("DM[10]=DEADBEEF",  32'hDEAD_BEEF,  readData);

## do_read(32'd200);

## check("DM[200]=CAFEF00D", 32'hCAFE_F00D,  readData);

## do_read(32'd255);

## check("DM[255]=0000FFFF", 32'h0000_FFFF,  readData);

## // ============================================================

## // 2. LED CONTROLLER  -  address[9:8] = 2'b01  (addresses 256-511)

## // ============================================================

## $display("\n[3] LED Controller - Writes");

## do_write(32'd256, 32'h0000_A5A5);   // first LED address

## @(posedge clk); #1;                 // extra cycle: led_reg latches

## check("LEDs=A5A5",        32'h0000_A5A5, {16'b0, leds});

## do_write(32'd300, 32'h0000_5A5A);

## @(posedge clk); #1;

## check("LEDs=5A5A",        32'h0000_5A5A, {16'b0, leds});

## do_write(32'd511, 32'h0000_1234);   // last LED address

## @(posedge clk); #1;

## check("LEDs=1234",        32'h0000_1234, {16'b0, leds});

## // Verify decoder exclusivity during an LED write

## @(negedge clk);

## address     = 32'd400;   // LED range

## writeEnable = 1'b1;

## readEnable  = 1'b0;

## #2;

## check("LED: LW=1",        32'd1, LEDWrite);

## check("LED: DmW=0",       32'd0, DataMemWrite);

## check("LED: DmR=0",       32'd0, DataMemRead);

## check("LED: SRE=0",       32'd0, SwitchReadEnable);

## @(posedge clk); #1;

## writeEnable = 1'b0;

## // ============================================================

## // 3. SWITCH INTERFACE  -  address[9:8] = 2'b10  (addresses 512-767)

## // ============================================================

## $display("\n[4] Switch Interface - Reads");

## switches = 16'hF0F0;

## do_read(32'd512);   // first Switch address → [9:8]=10

## check("SW[512]=F0F0", 32'h0000_F0F0, readData);

## check("SwitchRdEn hi", 32'd1,         SwitchReadEnable);

## check("SW: DmR=0",     32'd0,         DataMemRead);

## check("SW: DmW=0",     32'd0,         DataMemWrite);

## check("SW: LW =0",     32'd0,         LEDWrite);

## switches = 16'h0F0F;

## do_read(32'd600);   // mid Switch range

## check("SW[600]=0F0F", 32'h0000_0F0F, readData);

## check("SRE hi (600)", 32'd1,          SwitchReadEnable);

## switches = 16'hAAAA;

## do_read(32'd767);   // last Switch address

## check("SW[767]=AAAA", 32'h0000_AAAA, readData);

## check("SRE hi (767)", 32'd1,          SwitchReadEnable);

## switches = 16'h5555;

## do_read(32'd512);

## check("SW[512]=5555", 32'h0000_5555, readData);

## switches = 16'hFFFF;

## do_read(32'd700);

## check("SW[700]=FFFF", 32'h0000_FFFF, readData);

## // ============================================================

## // 4. UNUSED RANGE  -  address[9:8] = 2'b11  (addresses 768-1023)

## //    All decode outputs must remain low.

## // ============================================================

## $display("\n[5] Unused Address Range");

## @(negedge clk);

## address     = 32'd768;   // [9:8]=11 → no device

## readEnable  = 1'b1;

## writeEnable = 1'b0;

## #2;

## check("Unused: SRE=0", 32'd0, SwitchReadEnable);

## check("Unused: DmR=0", 32'd0, DataMemRead);

## check("Unused: LW =0", 32'd0, LEDWrite);

## check("Unused: DmW=0", 32'd0, DataMemWrite);

## check("Unused: rDat=0",32'd0, readData);

## @(posedge clk); #1;

## readEnable = 1'b0;

## @(negedge clk);

## address     = 32'd1023;  // [9:8]=11 → no device

## writeEnable = 1'b1;

## readEnable  = 1'b0;

## #2;

## check("Unused1023:DmW=0",32'd0, DataMemWrite);

## check("Unused1023:LW =0",32'd0, LEDWrite);

## @(posedge clk); #1;

## writeEnable = 1'b0;

## // ============================================================

## // 5. DECODER EXCLUSIVITY - no enables overlap for any device

## // ============================================================

## $display("\n[6] Decoder Exclusivity Sweep");

## // DataMem write

## @(negedge clk); address=32'd128; writeEnable=1; readEnable=0; #2;

## check("Excl DmW=1",  32'd1, DataMemWrite);

## check("Excl DmR=0",  32'd0, DataMemRead);

## check("Excl LW =0",  32'd0, LEDWrite);

## check("Excl SRE=0",  32'd0, SwitchReadEnable);

## @(posedge clk); #1; writeEnable=0;

## // DataMem read

## @(negedge clk); address=32'd128; readEnable=1; writeEnable=0; #2;

## check("Excl DmR=1",  32'd1, DataMemRead);

## check("Excl DmW=0",  32'd0, DataMemWrite);

## check("Excl LW =0",  32'd0, LEDWrite);

## check("Excl SRE=0",  32'd0, SwitchReadEnable);

## @(posedge clk); #1; readEnable=0;

## // LED write

## @(negedge clk); address=32'd350; writeEnable=1; readEnable=0; #2;

## check("Excl LW =1",  32'd1, LEDWrite);

## check("Excl DmW=0",  32'd0, DataMemWrite);

## check("Excl DmR=0",  32'd0, DataMemRead);

## check("Excl SRE=0",  32'd0, SwitchReadEnable);

## @(posedge clk); #1; writeEnable=0;

## // Switch read

## @(negedge clk); address=32'd650; readEnable=1; writeEnable=0; #2;

## check("Excl SRE=1",  32'd1, SwitchReadEnable);

## check("Excl DmW=0",  32'd0, DataMemWrite);

## check("Excl DmR=0",  32'd0, DataMemRead);

## check("Excl LW =0",  32'd0, LEDWrite);

## @(posedge clk); #1; readEnable=0;

## // ============================================================

## // Summary

## // ============================================================

## repeat(4) @(posedge clk);

## $display("\n====================================================");

## $display("  TOTAL:  PASS = %0d    FAIL = %0d", pass_cnt, fail_cnt);

## if (fail_cnt == 0)

## $display("  *** All tests passed! ***");

## else

## $display("  *** %0d test(s) FAILED - review above ***", fail_cnt);

## $display("====================================================\n");

## $stop;

## end

## endmodule

## Simulation waveforms showing correct operation\

## DATA MEMORY SECTION:

DATA MEMORY WRITE (40ns-100ns):

DATA MEMORY READ (100ns- 140ns)

Waveform explanations:

AT 100ns, all the values that were stored; will start showing at READ DATA. Where the yellow marker is places

LED SECTION:

As WRITE DATA has some value, so when led write rises, the value is shown on leds. After it falls back. As seen in the wave forms. When first ledwrite falls at 160ns, the led output can be seen same as writedata.

Same goes here

SWITCH SECTION:

As we put input in swiches, then when the READDATA turns 1, it basically replicates the value of switches, as can be seen in wave forms.

As here in this wave form we can see that switch read enable was zero. Readdata, did not get the value which was in switches.

Now when the switchreadenable becomes 1. It reads the value of switches as can be seen in the following waveforms.

Rest is also the same way

# Picture of full waveform of test bench

# Task 3: FPGA Synthesis and RTL Analysis

## RTL schematic screenshots (register file array, decoder, FSM, ALU connections)

## Demonstration of FPGA operation

## SW0  → writeData bit0  → LD0

## SW1  → writeData bit1  → LD1

## SW2  → writeData bit2  → LD2

## SW3  → writeData bit3  → LD3

## SW4  → writeData bit4  → LD4

## SW5  → writeData bit5  → LD5

## SW6  → writeData bit6  → LD6

## SW7  → writeData bit7  → LD7

## SW8  → address[8]  (device select LSB)

## SW9  → address[9]  (device select MSB)

## SW10 → readEnable

## SW11 → writeEnable

## SW12 → unused

## SW13 → unused

## SW14 → unused

## SW15 → unused

## Write data

Read data from switches. No led will turn on

Write data holds value when write enable is off. And switches are even changed

All LEDs will be OFF when:

Reset; press btnC anytime

SW11=DOWN: writeEnable off, nothing gets written

SW8=DOWN, SW9=DOWN, SW11=UP: memory region, LEDWrite never asserts

SW8=DOWN, SW9=UP, SW11=UP: switch region, LEDWrite never asserts

SW8=UP, SW9=UP, SW11=UP: invalid region 11, no device selected

SW8=UP, SW9=DOWN, SW11=UP, SW0-SW7=all DOWN: correct region but writing 0x00 to LEDs

Power on before any switch touched: register initializes to 0

# Assessment Rubric

Lab 8: Design and FPGA Implementation of Memory System with Address Decoding for a Single-Cycle RISC-V Processor

| Name: | Student ID: | Section: |
| --- | --- | --- |

Points Distribution

|  | Task No. | LR 2 (Code) | LR 5 (Results) | LR 11 (Design) |
| --- | --- | --- | --- | --- |
| In-Lab | Task 1 | - | - | /20 |
|  | Task 2 | /20 | /15 | - |
|  | Task 3 | /10 | /15 | - |
| Total Points |  | /30 | /30 | /20 |
| CLO Mapped |  | CLO 1 |  |  |

| Affective Domain Rubric |  | Points | CLO Mapped |
| --- | --- | --- | --- |
| AR7 | Report Submission & Git Upload | /10 & /10 | CLO 1 |

| CLO | Total Points | Points Obtained |
| --- | --- | --- |
| 1 | 100 |  |
| Total | 100 |  |

For description of different levels of the mapped rubrics, please refer to the Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics provided here.