# Syed Sualeh Abbas 10303,Mohsin Mujtaba, Zaid Naqvi

# Lab 9: Design and FPGA Implementation of the Control Path for a Single-Cycle RISC-V Processor

# Task 1: Developing the Control Path Truth Table

## A truth table for the main control unit (opcode as input, control signals as outputs).

A truth table for the ALU control unit (ALUOp, funct3, funct7 as inputs).

# Task 2: Verilog Description and Testbench Development

## Verilog source file for Main Control

## module MainControl (

## input  wire [6:0] opcode,

## output reg        RegWrite,

## output reg        ALUSrc,

## output reg        MemRead,

## output reg        MemWrite,

## output reg        MemtoReg,

## output reg        Branch,

## output reg  [1:0] ALUOp

## );

## // ── Opcode constants ────────────────────────────────────────────

## localparam R_TYPE = 7'b0110011;  // ADD SUB SLL SRL AND OR XOR

## localparam I_ALU  = 7'b0010011;  // ADDI

## localparam I_LOAD = 7'b0000011;  // LW LH LB

## localparam S_TYPE = 7'b0100011;  // SW SH SB

## localparam B_TYPE = 7'b1100011;  // BEQ

## always @(*) begin

## // ── Safe defaults (prevents unintended latches) ─────────────

## RegWrite = 1'b0;

## ALUSrc   = 1'b0;

## MemRead  = 1'b0;

## MemWrite = 1'b0;

## MemtoReg = 1'b0;

## Branch   = 1'b0;

## ALUOp    = 2'b00;

## case (opcode)

## R_TYPE: begin

## RegWrite = 1'b1;

## ALUSrc   = 1'b0;

## MemRead  = 1'b0;

## MemWrite = 1'b0;

## MemtoReg = 1'b0;

## Branch   = 1'b0;

## ALUOp    = 2'b10;

## end

## I_ALU: begin

## RegWrite = 1'b1;

## ALUSrc   = 1'b1;

## MemRead  = 1'b0;

## MemWrite = 1'b0;

## MemtoReg = 1'b0;

## Branch   = 1'b0;

## ALUOp    = 2'b10;

## end

## I_LOAD: begin

## RegWrite = 1'b1;

## ALUSrc   = 1'b1;

## MemRead  = 1'b1;

## MemWrite = 1'b0;

## MemtoReg = 1'b1;

## Branch   = 1'b0;

## ALUOp    = 2'b00;

## end

## S_TYPE: begin

## RegWrite = 1'b0;

## ALUSrc   = 1'b1;

## MemRead  = 1'b0;

## MemWrite = 1'b1;

## MemtoReg = 1'b0;  // don't-care → safe 0

## Branch   = 1'b0;

## ALUOp    = 2'b00;

## end

## B_TYPE: begin

## RegWrite = 1'b0;

## ALUSrc   = 1'b0;

## MemRead  = 1'b0;

## MemWrite = 1'b0;

## MemtoReg = 1'b0;  // don't-care → safe 0

## Branch   = 1'b1;

## ALUOp    = 2'b01;

## end

## default: begin

## RegWrite = 1'b0;

## ALUSrc   = 1'b0;

## MemRead  = 1'b0;

## MemWrite = 1'b0;

## MemtoReg = 1'b0;

## Branch   = 1'b0;

## ALUOp    = 2'b00;

## end

## endcase

## end

## endmodule

## Verilog source file for ALU Control

## module ALUControl (

## input  wire [1:0] ALUOp,    // From MainControl

## input  wire [2:0] funct3,   // Instruction bits [14:12]

## input  wire [6:0] funct7,   // Instruction bits [31:25]

## output reg  [3:0] ALUControl // To ALU32bit

## );

## always @(*) begin

## case (ALUOp)

## // ----------------------------------------------------------

## // ALUOp = 00 → Force ADD  (LW, LH, LB, SW, SH, SB)

## //   Address calculation: rs1 + sign_ext(imm)

## // ----------------------------------------------------------

## 2'b00: ALUControl = 4'b0010;   // ADD

## // ----------------------------------------------------------

## // ALUOp = 01 → Force SUB  (BEQ)

## //   rs1 - rs2; if Zero=1 branch is taken

## // ----------------------------------------------------------

## 2'b01: ALUControl = 4'b0110;   // SUB

## // ----------------------------------------------------------

## // ALUOp = 10 → Decode funct3 / funct7

## //   Covers all R-type instructions and ADDI

## // ----------------------------------------------------------

## 2'b10: begin

## case (funct3)

## 3'b000: begin   // ADD (funct7=0000000) / SUB (funct7=0100000) / ADDI

## if (funct7 == 7'b0100000)

## ALUControl = 4'b0110;  // SUB

## else

## ALUControl = 4'b0010;  // ADD / ADDI

## end

## 3'b001: ALUControl = 4'b0100;  // SLL

## 3'b101: ALUControl = 4'b0101;  // SRL

## 3'b111: ALUControl = 4'b0000;  // AND

## 3'b110: ALUControl = 4'b0001;  // OR

## 3'b100: ALUControl = 4'b0011;  // XOR

## default: ALUControl = 4'b0010; // safe default → ADD

## endcase

## end

## // ----------------------------------------------------------

## // Default: force ADD (safe)

## // ----------------------------------------------------------

## default: ALUControl = 4'b0010;

## endcase

## end

## endmodule

## Testbench

## module tb_control_path;

## // ── MainControl ports ────────────────────────────────────────────

## reg  [6:0] opcode;

## wire       RegWrite, ALUSrc, MemRead, MemWrite, MemtoReg, Branch;

## wire [1:0] ALUOp;

## // ── ALUControl ports ─────────────────────────────────────────────

## reg  [2:0] funct3;

## reg  [6:0] funct7;

## wire [3:0] ALUControl;

## // ── ALU32bit ports ────────────────────────────────────────────────

## reg  [31:0] A, B;

## wire [31:0] ALUResult;

## wire        Zero;

## // ── Error counter ─────────────────────────────────────────────────

## integer errors;

## // ── DUT instantiations ────────────────────────────────────────────

## MainControl u_main (

## .opcode   (opcode),

## .RegWrite (RegWrite),

## .ALUSrc   (ALUSrc),

## .MemRead  (MemRead),

## .MemWrite (MemWrite),

## .MemtoReg (MemtoReg),

## .Branch   (Branch),

## .ALUOp    (ALUOp)

## );

## ALUControl u_alu_ctrl (

## .ALUOp      (ALUOp),

## .funct3     (funct3),

## .funct7     (funct7),

## .ALUControl (ALUControl)

## );

## ALU32bit u_alu (

## .A          (A),

## .B          (B),

## .ALUControl (ALUControl),

## .ALUResult  (ALUResult),

## .Zero       (Zero)

## );

## // ── Helper task: check control signals ───────────────────────────

## task check_ctrl;

## input [63:0] tag;

## input exp_RegWrite, exp_ALUSrc, exp_MemRead, exp_MemWrite;

## input exp_MemtoReg, exp_Branch;

## input [1:0]  exp_ALUOp;

## input [3:0]  exp_ALUControl;

## begin

## #5;

## if (RegWrite    !== exp_RegWrite)    begin $display("FAIL [%s] RegWrite   : got %b exp %b", tag, RegWrite,   exp_RegWrite);   errors=errors+1; end

## if (ALUSrc      !== exp_ALUSrc)      begin $display("FAIL [%s] ALUSrc     : got %b exp %b", tag, ALUSrc,     exp_ALUSrc);     errors=errors+1; end

## if (MemRead     !== exp_MemRead)     begin $display("FAIL [%s] MemRead    : got %b exp %b", tag, MemRead,    exp_MemRead);    errors=errors+1; end

## if (MemWrite    !== exp_MemWrite)    begin $display("FAIL [%s] MemWrite   : got %b exp %b", tag, MemWrite,   exp_MemWrite);   errors=errors+1; end

## if (MemtoReg    !== exp_MemtoReg)    begin $display("FAIL [%s] MemtoReg   : got %b exp %b", tag, MemtoReg,   exp_MemtoReg);   errors=errors+1; end

## if (Branch      !== exp_Branch)      begin $display("FAIL [%s] Branch     : got %b exp %b", tag, Branch,     exp_Branch);     errors=errors+1; end

## if (ALUOp       !== exp_ALUOp)       begin $display("FAIL [%s] ALUOp      : got %b exp %b", tag, ALUOp,      exp_ALUOp);      errors=errors+1; end

## if (ALUControl  !== exp_ALUControl)  begin $display("FAIL [%s] ALUControl : got %b exp %b", tag, ALUControl, exp_ALUControl); errors=errors+1; end

## end

## endtask

## // ── Helper task: check ALU arithmetic result ──────────────────────

## task check_alu_result;

## input [63:0]  tag;

## input [31:0]  exp_result;

## input         exp_zero;

## begin

## if (ALUResult !== exp_result) begin

## $display("FAIL [%s] ALUResult: got %0d exp %0d", tag, ALUResult, exp_result);

## errors = errors + 1;

## end else begin

## $display("PASS [%s] ALUResult=%0d Zero=%b ALUControl=%b", tag, ALUResult, Zero, ALUControl);

## end

## if (Zero !== exp_zero) begin

## $display("FAIL [%s] Zero flag: got %b exp %b", tag, Zero, exp_zero);

## errors = errors + 1;

## end

## end

## endtask

## // ── Stimulus ─────────────────────────────────────────────────────

## initial begin

## errors = 0;

## $dumpfile("waveform.vcd");

## $dumpvars(0, tb_control_path);

## A = 32'd20; B = 32'd7;

## $display("==============================================");

## $display("  CONTROL PATH VERIFICATION");

## $display("  ALU32bit encoding used for ALUControl");

## $display("==============================================");

## // ── R-TYPE ADD ─────────────────────────────────────────────

## // opcode=0110011 funct3=000 funct7=0000000

## // Expect: RegWrite=1 ALUSrc=0 MemRead=0 MemWrite=0 MemtoReg=0 Branch=0 ALUOp=10

## // ALUControl=0010 (ADD), ALUResult=27, Zero=0

## opcode=7'b0110011; funct3=3'b000; funct7=7'b0000000;

## check_ctrl("ADD     ",1,0,0,0,0,0,2'b10,4'b0010);

## check_alu_result("ADD     ",32'd27,1'b0);

## // ── R-TYPE SUB ─────────────────────────────────────────────

## // opcode=0110011 funct3=000 funct7=0100000

## // ALUControl=0110 (SUB), ALUResult=13, Zero=0

## opcode=7'b0110011; funct3=3'b000; funct7=7'b0100000;

## check_ctrl("SUB     ",1,0,0,0,0,0,2'b10,4'b0110);

## check_alu_result("SUB     ",32'd13,1'b0);

## // ── R-TYPE SUB equal (Zero flag test) ──────────────────────

## A=32'd10; B=32'd10;

## opcode=7'b0110011; funct3=3'b000; funct7=7'b0100000;

## check_alu_result("SUB_ZERO",32'd0,1'b1);

## A=32'd20; B=32'd7;

## // ── R-TYPE SLL ─────────────────────────────────────────────

## // ALUControl=0100 (SLL), ALUResult = 20 << 7 = 2560

## opcode=7'b0110011; funct3=3'b001; funct7=7'b0000000;

## check_ctrl("SLL     ",1,0,0,0,0,0,2'b10,4'b0100);

## check_alu_result("SLL     ",32'd2560,1'b0);

## // ── R-TYPE SRL ─────────────────────────────────────────────

## // ALUControl=0101 (SRL), ALUResult = 20 >> 7 = 0

## opcode=7'b0110011; funct3=3'b101; funct7=7'b0000000;

## check_ctrl("SRL     ",1,0,0,0,0,0,2'b10,4'b0101);

## check_alu_result("SRL     ",32'd0,1'b1);

## // ── R-TYPE AND ─────────────────────────────────────────────

## // ALUControl=0000 (AND), ALUResult = 20&7 = 4

## opcode=7'b0110011; funct3=3'b111; funct7=7'b0000000;

## check_ctrl("AND     ",1,0,0,0,0,0,2'b10,4'b0000);

## check_alu_result("AND     ",32'd4,1'b0);

## // ── R-TYPE OR ──────────────────────────────────────────────

## // ALUControl=0001 (OR), ALUResult = 20|7 = 23

## opcode=7'b0110011; funct3=3'b110; funct7=7'b0000000;

## check_ctrl("OR      ",1,0,0,0,0,0,2'b10,4'b0001);

## check_alu_result("OR      ",32'd23,1'b0);

## // ── R-TYPE XOR ─────────────────────────────────────────────

## // ALUControl=0011 (XOR), ALUResult = 20^7 = 19

## opcode=7'b0110011; funct3=3'b100; funct7=7'b0000000;

## check_ctrl("XOR     ",1,0,0,0,0,0,2'b10,4'b0011);

## check_alu_result("XOR     ",32'd19,1'b0);

## // ── ADDI ───────────────────────────────────────────────────

## // opcode=0010011 funct3=000 funct7=X (imm bits - treated as 0 here)

## // ALUControl=0010 (ADD), ALUResult=27

## opcode=7'b0010011; funct3=3'b000; funct7=7'b0000000;

## check_ctrl("ADDI    ",1,1,0,0,0,0,2'b10,4'b0010);

## check_alu_result("ADDI    ",32'd27,1'b0);

## // ── LOAD (LW) ──────────────────────────────────────────────

## // opcode=0000011

## // ALUOp=00 → ALUControl=0010 (ADD, address = rs1 + imm)

## // RegWrite=1 MemRead=1 ALUSrc=1 MemtoReg=1

## opcode=7'b0000011; funct3=3'b010; funct7=7'b0000000;

## check_ctrl("LW      ",1,1,1,0,1,0,2'b00,4'b0010);

## check_alu_result("LW_ADDR ",32'd27,1'b0);  // address = 20+7

## // ── STORE (SW) ─────────────────────────────────────────────

## // opcode=0100011

## // ALUOp=00 → ALUControl=0010 (ADD, address = rs1 + imm)

## // RegWrite=0 MemWrite=1 ALUSrc=1

## opcode=7'b0100011; funct3=3'b010; funct7=7'b0000000;

## check_ctrl("SW      ",0,1,0,1,0,0,2'b00,4'b0010);

## check_alu_result("SW_ADDR ",32'd27,1'b0);

## // ── BEQ (not equal) ────────────────────────────────────────

## // opcode=1100011

## // ALUOp=01 → ALUControl=0110 (SUB), Zero=0 → branch NOT taken

## opcode=7'b1100011; funct3=3'b000; funct7=7'b0000000;

## check_ctrl("BEQ_NE  ",0,0,0,0,0,1,2'b01,4'b0110);

## check_alu_result("BEQ_NE  ",32'd13,1'b0);  // 20-7=13, Zero=0

## // ── BEQ (equal - branch taken) ─────────────────────────────

## A=32'd15; B=32'd15;

## opcode=7'b1100011; funct3=3'b000; funct7=7'b0000000;

## check_alu_result("BEQ_EQ  ",32'd0,1'b1);   // 15-15=0, Zero=1 → branch taken

## // ── Summary ────────────────────────────────────────────────

## #10;

## $display("==============================================");

## if (errors == 0)

## $display("  ALL %0d TESTS PASSED", 24);

## else

## $display("  %0d TEST(S) FAILED", errors);

## $display("==============================================");

## $finish;

## end

## endmodule

## Simulation waveform screenshots highlighting all control signal transitions.

Waveforms and explanation:

### Segment 1: opcode = 0x33 → R-TYPE (0–~25 ns)

### This covers ADD, SUB, SLL, SRL, AND, OR, XOR — funct3 cycles through 0,1,5,7,6,4

ADD:

SUB:

SLL:

SRL:

AND:

OR:

XOR:

Segment 2: opcode = 0x13 → ADDI (~25–33 ns)

Segment 3: opcode = 0x03 → LOAD (~33–38 ns)

Segment 4: opcode = 0x23 → STORE (~38–46 ns)

Segment 5: opcode = 0x63 → BEQ (~46–60 ns)

# Task 3: FPGA Synthesis and RTL Analysis

## Deliverables

Note: The switch interface, button debouncing module, and LED interface must be re used from Lab 5.

## RTL schematic screenshots

## Demonstration of FPGA operation

## Led mapping:

| ## LEDS |  |
| --- | --- |
| ## 0 | ## Register Write |
| ## 1 | ## Mem read |
| ## 2 | ## Mem Write |
| ## 3 | ## Alu SRC |
| ## 4 | ## Mem to Reg |
| ## 5 |  |
| ## 6 - 9 | ## ALU control |
| ## 10-11 | ## ALU op |

SW 0-6: OPCODE

SW 7-9: Fuct 3

SW 10: Function 7

REGISTER WRITE:

LOAD REGISTER:

# Assessment Rubric

Lab 9: Design and FPGA Implementation of the Control Path for a Single-Cycle RISC-V Processor

| Name: | Student ID: | Section: |
| --- | --- | --- |

Points Distribution

|  | Task No. | LR 2 (Code) | LR 5 (Results) | LR 10 (Analysis) |
| --- | --- | --- | --- | --- |
| In-Lab | Task 1 | - | - | /20 |
|  | Task 2 | /20 | /15 | - |
|  | Task 3 | /10 | /15 | - |
| Total Points |  | /30 | /30 | /20 |
| CLO Mapped |  | CLO 1 |  |  |

| Affective Domain Rubric |  | Points | CLO Mapped |
| --- | --- | --- | --- |
| AR7 | Report Submission & Git Upload | /10 & /10 | CLO 1 |

| CLO | Total Points | Points Obtained |
| --- | --- | --- |
| 1 | 100 |  |
| Total | 100 |  |

For description of different levels of the mapped rubrics, please refer to the Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics