LAB 08:

TASK 1:

/* USER CODE BEGIN PD */

/* ── Gyroscope Register Addresses ─────────────────────────────────────── */

#define GYRO_WHO_AM_I_ADDR   0x0F   /* Task 1: WHO_AM_I register          */

#define I3G4250D_WHO_AM_I    0xD3   /* Task 1: Expected ID - blue board   */

#define L3GD20_WHO_AM_I      0xD4   /* Task 1: Expected ID - green board  */

#define CTRL_REG1            0x20

#define CTRL_REG1_VAL        0x8F   /* PD=1, ODR=400Hz, Zen=Yen=Xen=1    */

#define CTRL_REG4            0x23

#define CTRL_REG4_VAL        0x00   /* FS = ±245 dps (default)            */

#define OUT_TEMP             0x26   /* Temperature output register        */

#define OUT_X_L              0x28   /* X-axis low byte (auto-inc from here)*/

/* ── SPI Command Bits ──────────────────────────────────────────────────── */

#define SPI_READ             0x80   /* Bit7=1 => read                     */

#define SPI_AUTO_INC         0x40   /* Bit6=1 => auto-increment address   */

/* ── Chip Select Pin ───────────────────────────────────────────────────── */

#define GYRO_CS_PIN          GPIO_PIN_3

#define GYRO_CS_PORT         GPIOE

/* USER CODE END PD */

/* USER CODE BEGIN PM */

#define GYRO_CS_LOW()   HAL_GPIO_WritePin(GYRO_CS_PORT, GYRO_CS_PIN, GPIO_PIN_RESET)

#define GYRO_CS_HIGH()  HAL_GPIO_WritePin(GYRO_CS_PORT, GYRO_CS_PIN, GPIO_PIN_SET)

/* USER CODE END PM */

I2C_HandleTypeDef  hi2c1;

SPI_HandleTypeDef  hspi1;

UART_HandleTypeDef huart2;

PCD_HandleTypeDef  hpcd_USB_FS;

/* USER CODE BEGIN PV */

static uint8_t Gyro_ReadRegister(uint8_t reg_addr);

Usage in USER CODE BEGIN 2:

uint8_t id = Gyro_ReadRegister(GYRO_WHO_AM_I_ADDR);

sprintf(uart_buf, "WHO_AM_I = 0x%02X\r\n", id);

HAL_UART_Transmit(&huart2, (uint8_t*)uart_buf, strlen(uart_buf), 100);

// static uint8_t Gyro_ReadRegister(uint8_t reg_addr)

// {

//     uint8_t cmd     = SPI_READ | (reg_addr & 0x3F);

//     uint8_t rx_data = 0x00;

//     GYRO_CS_LOW();

//     HAL_SPI_Transmit(&hspi1, &cmd, 1, 100);

//     HAL_SPI_Receive(&hspi1, &rx_data, 1, 100);

//     GYRO_CS_HIGH();

//     return rx_data;

// }

// ── Task 1 (commented out) ─────────────────────────────────────────

// uint8_t who_am_i = Gyro_ReadRegister(GYRO_WHO_AM_I_ADDR);

// if (who_am_i == I3G4250D_WHO_AM_I)

//     sprintf(uart_buf, "WHO_AM_I = 0x%02X --> I3G4250D (PASS)\r\n", who_am_i);

// else if (who_am_i == L3GD20_WHO_AM_I)

//     sprintf(uart_buf, "WHO_AM_I = 0x%02X --> L3GD20 (PASS)\r\n", who_am_i);

// else

//     sprintf(uart_buf, "WHO_AM_I = 0x%02X --> UNKNOWN\r\n", who_am_i);

// HAL_UART_Transmit(&huart2, (uint8_t*)uart_buf, strlen(uart_buf), 100);

RESULTS:

TASK 2:

// typedef enum {

//     SPI_STATE_IDLE = 0,

//     SPI_STATE_TX_DONE,

//     SPI_STATE_RX_DONE

// } SPI_State_t;

// volatile SPI_State_t spiState = SPI_STATE_IDLE;

// uint8_t t2_tx_buf[1];

// uint8_t t2_rx_buf[1];

// volatile int8_t t2_temperature = 0;

// void gyro_read_temp_start(void);

// void HAL_SPI_TxCpltCallback(SPI_HandleTypeDef *hspi);

// void HAL_SPI_RxCpltCallback(SPI_HandleTypeDef *hspi);

// void gyro_read_temp_start(void)

// {

//     t2_tx_buf[0] = SPI_READ | OUT_TEMP;

//     spiState     = SPI_STATE_IDLE;

//     GYRO_CS_LOW();

//     HAL_SPI_Transmit_IT(&hspi1, t2_tx_buf, 1);

// }

//

// void HAL_SPI_TxCpltCallback(SPI_HandleTypeDef *hspi)

// {

//     if (hspi->Instance == SPI1)

//     {

//         spiState = SPI_STATE_TX_DONE;

//         HAL_SPI_Receive_IT(&hspi1, t2_rx_buf, 1);

//     }

// }

//

// void HAL_SPI_RxCpltCallback(SPI_HandleTypeDef *hspi)

// {

//     if (hspi->Instance == SPI1)

//     {

//         GYRO_CS_HIGH();

//         t2_temperature = (int8_t)t2_rx_buf[0];

//         spiState       = SPI_STATE_RX_DONE;

//     }}

// GYRO_CS_HIGH();

// HAL_Delay(10);

// gyro_init_t2();   /* Task 2 used its own init writing only CTRL_REG1=0x0F */

// HAL_Delay(100);

// -- while loop was: gyro_read_temp_start(); wait spiState; transmit; delay

RESULTS:

TASK 3:

static uint8_t rx_buf[6];

static int8_t  temp_raw;

static int16_t x_raw, y_raw, z_raw;

static char    uart_buf[64];

/* USER CODE END PV */

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_I2C1_Init(void);

static void MX_SPI1_Init(void);

static void MX_USART2_UART_Init(void);

static void MX_USB_PCD_Init(void);

static void    gyro_write_reg(uint8_t reg, uint8_t val);

static uint8_t gyro_read_reg(uint8_t reg);

static void    gyro_init(void);

static void    gyro_read_all(void);

static int     write_dps(char *buf, int32_t raw);

static void gyro_write_reg(uint8_t reg, uint8_t val)

{

uint8_t tx[2] = { reg & 0x3F, val };

GYRO_CS_LOW();

HAL_SPI_Transmit(&hspi1, tx, 2, HAL_MAX_DELAY);

GYRO_CS_HIGH();

}

static uint8_t gyro_read_reg(uint8_t reg)

{

uint8_t cmd  = SPI_READ | (reg & 0x3F);

uint8_t data = 0;

GYRO_CS_LOW();

HAL_SPI_Transmit(&hspi1, &cmd,  1, HAL_MAX_DELAY);

HAL_SPI_Receive (&hspi1, &data, 1, HAL_MAX_DELAY);

GYRO_CS_HIGH();

return data;

}

static void gyro_init(void)

{

gyro_write_reg(CTRL_REG1, CTRL_REG1_VAL);  /* Power on, enable axes  */

gyro_write_reg(CTRL_REG4, CTRL_REG4_VAL);  /* ±245 dps full scale    */

}

static void gyro_read_all(void)

{

/* Read temperature */

temp_raw = (int8_t)gyro_read_reg(OUT_TEMP);

/* Burst read 6 bytes: OUT_X_L through OUT_Z_H using auto-increment */

uint8_t cmd = SPI_READ | SPI_AUTO_INC | OUT_X_L;  /* 0xE8 */

GYRO_CS_LOW();

HAL_SPI_Transmit(&hspi1, &cmd,   1, HAL_MAX_DELAY);

HAL_SPI_Receive (&hspi1, rx_buf, 6, HAL_MAX_DELAY);

GYRO_CS_HIGH();

/* Merge low and high bytes (little-endian) */

x_raw = (int16_t)((rx_buf[1] << 8) | rx_buf[0]);

y_raw = (int16_t)((rx_buf[3] << 8) | rx_buf[2]);

z_raw = (int16_t)((rx_buf[5] << 8) | rx_buf[4]);

}

static int write_dps(char *buf, int32_t raw)

{

int32_t scaled   = (raw * 875L) / 100L;  /* units of 0.01 dps */

int     negative = (scaled < 0);

if (negative) scaled = -scaled;

int32_t whole = scaled / 100;

int32_t frac  = scaled % 100;

if (negative)

return sprintf(buf, "-%ld.%02ld", (long)whole, (long)frac);

else

return sprintf(buf,  "%ld.%02ld", (long)whole, (long)frac);

}

int main(void)

{

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_I2C1_Init();

MX_SPI1_Init();

MX_USART2_UART_Init();

MX_USB_PCD_Init();

GYRO_CS_HIGH();

HAL_Delay(10);

gyro_init();

HAL_Delay(100);

/* USER CODE END 2 */

/* USER CODE BEGIN WHILE */

while (1)

{

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

gyro_read_all();

char xs[12], ys[12], zs[12];

write_dps(xs, x_raw);

write_dps(ys, y_raw);

write_dps(zs, z_raw);

int len = sprintf(uart_buf, "%d, %s, %s, %s\n",

(int)temp_raw, xs, ys, zs);

HAL_UART_Transmit(&huart2, (uint8_t *)uart_buf, (uint16_t)len, HAL_MAX_DELAY);

HAL_Delay(100);

}

/* USER CODE END 3 */

}

RESULT: